# ADVANCED MATHEMATICS REPORT

Advanced Engineering Mathematics Report

Project: Darvycom Brisbane Engineering Laboratory Network and DSP Upgrade
Unit: ICTTEN827 – Produce engineering solutions using advanced engineering mathematics
Location: Blocks A and B, Brisbane Engineering Laboratory
Project Manager: Leonardo Matias Ciarcia
Document ID: C6-MATH-01
Version: 1.0
Date: 03 March 2026
Project baseline: 24 weeks / AUD 184,800

## 1. Executive summary

This report demonstrates the use of advanced engineering mathematics to support the Darvycom network and digital signal processing solution. Mathematical methods were used to calculate network capacity, VoIP bandwidth, subnet sizes, queue performance, signal frequency components, convolution results, signal-to-noise ratio and control-system behaviour.

The calculations support the selected Network 3 design, which includes 20 computers, 20 IP phones, 40 headsets, two data servers, a Cisco 2911 voice server, a VPN router, four managed PoE switches, wireless access points, video conferencing, VLAN segmentation, QoS and PRTG monitoring.

The results show that the proposed design has sufficient capacity under normal operating conditions. They also demonstrate why QoS, queue thresholds and rate control are necessary when traffic approaches the available link capacity.

## 2. Purpose and scope

The purpose of this report is to:

- apply engineering mathematics to the Darvycom telecommunications solution;

- calculate network bandwidth and utilisation;

- determine suitable subnet sizes;

- analyse network queues and congestion;

- apply Fourier transform and convolution calculations;

- calculate signal-to-noise ratio improvement;

- use differential equations and Laplace transforms to model system behaviour;

- support engineering decisions with numerical results.

The calculations are based on the approved Network 3 design and the queue-based simulation control system.

## 3. Mathematical methods used

| Mathematical method | Application to Darvycom |
| --- | --- |
| Arithmetic and percentages | Bandwidth, headroom and utilisation |
| Binary and subnet mathematics | VLAN subnet selection |
| Matrix calculations | Distribution of traffic between links |
| Complex numbers | Frequency response of a DSP filter |
| Fourier transform | Identification of frequency components |
| Convolution | Calculation of a filtered output signal |
| Logarithms | Signal-to-noise ratio in decibels |
| Queue theory | Delay, utilisation and stability |
| Differential equations | Change in queue length over time |
| Laplace transforms | Dynamic response of the congestion controller |
| Numerical software | MATLAB calculation and simulation |

## 4. Network bandwidth calculation

### 4.1 VoIP bandwidth

The estimated bandwidth for one G.711 VoIP call, including protocol overhead, is 87.2 kbit/s.

Formula:

Total voice bandwidth = Number of calls × Bandwidth per call

Calculation:

Total voice bandwidth = 20 × 87.2 kbit/s
Total voice bandwidth = 1,744 kbit/s
Total voice bandwidth = 1.744 Mbit/s

A 25% design allowance is included for signalling, traffic variation and future growth.

Formula:

Planned voice capacity = Voice bandwidth × 1.25

Calculation:

Planned voice capacity = 1.744 × 1.25
Planned voice capacity = 2.18 Mbit/s

Therefore, at least 2.18 Mbit/s should be reserved for voice traffic.

### 4.2 Total planned traffic

The representative planned load is:

| Traffic category | Planned bandwidth |
| --- | --- |
| Voice, including headroom | 2.18 Mbit/s |
| Video conferencing | 24.00 Mbit/s |
| General data and applications | 40.00 Mbit/s |
| Total | 66.18 Mbit/s |

Formula:

Utilisation = Total traffic / Link capacity × 100

For a 100 Mbit/s test link:

Utilisation = 66.18 / 100 × 100
Utilisation = 66.18%

For a 1 Gbit/s production uplink:

Utilisation = 66.18 / 1000 × 100
Utilisation = 6.618%

The production Gigabit uplinks provide significant capacity. The 100 Mbit/s link is used in the queue simulation because it makes congestion and controller behaviour easier to demonstrate.

## 5. Subnet mathematics

The number of usable IPv4 host addresses is calculated using:

Usable hosts = 2^h − 2

Where:

- h is the number of host bits;

- two addresses are removed for the network and broadcast addresses.

For a VLAN containing 20 endpoints:

2^4 − 2 = 14 usable addresses

This is insufficient.

2^5 − 2 = 30 usable addresses

Therefore, five host bits are required.

Prefix length = 32 − 5
Prefix length = /27

A /27 subnet provides 30 usable host addresses and is appropriate for the 20-device computer and IP phone VLANs.

For smaller server or management segments:

2^4 − 2 = 14 usable addresses

Therefore, a /28 subnet is suitable where no more than 14 addresses are required.

## 6. Matrix calculation for traffic distribution

A simple matrix was used to model the distribution of 66.18 Mbit/s between two logical paths. The preferred path carries twice as much traffic as the secondary path.

Equations:

x₁ + x₂ = 66.18
x₁ − 2x₂ = 0

Matrix form:

A x = b

A = [1 1; 1 −2]

x = [x₁; x₂]

b = [66.18; 0]

Solution:

x₂ = 22.06 Mbit/s
x₁ = 44.12 Mbit/s

Check:

44.12 + 22.06 = 66.18 Mbit/s

This calculation can support load-balancing or traffic-engineering decisions when two paths are available.

## 7. Queue theory calculations

### 7.1 Service rate

For a 100 Mbit/s link and an average packet size of 1,500 bytes:

Formula:

μ = C / (L × 8)

Where:

- μ = service rate in packets per second;

- C = link capacity in bits per second;

- L = packet size in bytes.

Calculation:

μ = 100,000,000 / (1,500 × 8)
μ = 8,333.33 packets/s

### 7.2 Queue utilisation

Formula:

ρ = λ / μ

Where:

- ρ = queue utilisation;

- λ = packet arrival rate;

- μ = packet service rate.

### 7.3 M/M/1 delay

For a stable M/M/1 queue:

W = 1 / (μ − λ)

Wq = λ / [μ(μ − λ)]

Where:

- W = average total time in the system;

- Wq = average waiting time in the queue.

| Scenario | λ | ρ | W | Queue condition |
| --- | --- | --- | --- | --- |
| Normal | 5,000 pkt/s | 0.60 | 0.30 ms | Stable |
| Busy | 7,500 pkt/s | 0.90 | 1.20 ms | Stable but sensitive |
| Overload | 9,000 pkt/s | 1.08 | Not finite | Unstable without control |
| Recovery | 4,000 pkt/s | 0.48 | 0.231 ms | Stable |

When λ is equal to or greater than μ, the basic queue is unstable. Packets arrive as fast as, or faster than, they can be transmitted. Rate limiting, packet dropping or additional capacity is therefore required.

## 8. Differential equation for queue behaviour

The change in queue length can be modelled using:

dq(t)/dt = λ(t) − μ(t)

Where:

- q(t) is the queue length;

- λ(t) is the packet arrival rate;

- μ(t) is the packet service rate.

For the overload scenario:

λ = 9,000 packets/s
μ = 8,333 packets/s

dq/dt = 9,000 − 8,333
dq/dt = 667 packets/s

If the queue initially contains 300 packets and has a 1,000-packet capacity:

Time to reach capacity = (1,000 − 300) / 667
Time to reach capacity = approximately 1.05 seconds

For recovery:

λ = 4,000 packets/s
μ = 8,333 packets/s

dq/dt = 4,000 − 8,333
dq/dt = −4,333 packets/s

Time to clear a 1,000-packet queue:

Time = 1,000 / 4,333
Time = approximately 0.231 seconds

The negative result for dq/dt means that the queue is decreasing.

## 9. Laplace transform and controller response

A first-order control-system model can represent the gradual restoration of data traffic after congestion.

Differential equation:

τ dy(t)/dt + y(t) = K u(t)

Transfer function:

G(s) = Y(s) / U(s) = K / (τs + 1)

Using:

K = 1
τ = 2 seconds

The unit-step response is:

y(t) = 1 − e^(−t/2)

At t = 2 seconds:

y(2) = 1 − e^(−1)
y(2) = 0.632

Therefore, the controller restores approximately 63.2% of the normal data allocation after two seconds.

At t = 6 seconds:

y(6) = 1 − e^(−3)
y(6) = approximately 0.950

The controller restores approximately 95% after six seconds. This gradual response reduces rapid switching between congestion states.

## 10. Fourier transform calculation

The discrete Fourier transform is:

X[k] = Σ x[n]e^(−j2πkn/N)

The frequency resolution is:

Δf = Fs / N

For:

Fs = 8,000 Hz
N = 1,024 samples

Calculation:

Δf = 8,000 / 1,024
Δf = 7.8125 Hz

A 1,000 Hz component appears at:

k = f / Δf
k = 1,000 / 7.8125
k = 128

The FFT should therefore show a peak near bin 128 for a 1 kHz input signal.

## 11. Convolution calculation

Input sequence:

x[n] = [1, 2, 1]

Impulse response:

h[n] = [1, 0.5]

Formula:

y[n] = x[n] * h[n]

Calculation:

y[0] = 1 × 1 = 1
y[1] = (2 × 1) + (1 × 0.5) = 2.5
y[2] = (1 × 1) + (2 × 0.5) = 2
y[3] = 1 × 0.5 = 0.5

Result:

y[n] = [1, 2.5, 2, 0.5]

MATLAB verification:

x = [1 2 1];

h = [1 0.5];

y = conv(x,h)

Expected output:

y =

1.0000    2.5000    2.0000    0.5000

## 12. Signal-to-noise ratio

Formula:

SNRdB = 10 log10(Psignal / Pnoise)

Before filtering:

Psignal = 0.01 W
Pnoise = 0.0001 W

SNRbefore = 10 log10(0.01 / 0.0001)
SNRbefore = 20 dB

After filtering:

Pnoise = 0.000025 W

SNRafter = 10 log10(0.01 / 0.000025)
SNRafter = 26.02 dB

Improvement:

SNR improvement = 26.02 − 20
SNR improvement = 6.02 dB

This result demonstrates that reducing noise power improves the receiving sensitivity and quality of the communication signal.

## 13. Engineering decisions

The mathematical analysis supports the following decisions:

- use /27 subnets for VLANs with approximately 20 endpoints;

- use /28 subnets for smaller server and management segments;

- reserve at least 2.18 Mbit/s for the 20 VoIP calls;

- use Gigabit uplinks in Network 3;

- implement voice, video and data queues;

- activate congestion control before utilisation reaches 100%;

- protect voice and video during traffic overload;

- use DSP filtering to improve SNR;

- verify mathematical calculations using MATLAB.

## 14. Conclusion

Advanced engineering mathematics was successfully applied to the Darvycom solution. The calculations validated bandwidth, subnetting, queue stability, DSP behaviour, SNR improvement and controller response. The results support the selection of VLAN segmentation, Gigabit uplinks, QoS, controlled queues and DSP filtering for the final Network 3 solution.