Advanced Engineering Mathematics Report

Project: Darvycom Brisbane Engineering Laboratory Network & DSP Upgrade

Location: Blocks A and B, Brisbane Engineering Laboratory

Project Manager: Leonardo Matias Ciarcia

Document ID: C6-MATH-01

Version: 1.0

Prepared: 15 March 2026

Baseline: 24 weeks / AUD 184,800

# 1. Purpose

This report demonstrates advanced engineering mathematics applied to multiple Darvycom solutions: IP subnet sizing, traffic capacity, latency and queueing, DSP convolution/Fourier resolution and service availability. Each calculation is tied to a design or acceptance decision rather than presented as isolated theory.

# 2. Solution A - Addressing and subnet capacity

A subnet with prefix /p contains 2^(32-p) IPv4 addresses. For a conventional subnet, usable host capacity is 2^(32-p) - 2. A /27 has 32 addresses and 30 usable hosts; it is suitable for a 20-device PC VLAN or 20-phone voice VLAN with room for gateways and modest growth. A /28 has 16 addresses and 14 usable hosts; it is suitable for smaller server, management or video segments when the expected device count remains below that limit.

| Use | Required hosts | Selected size | Usable hosts | Headroom |
| --- | --- | --- | --- | --- |
| PC/data VLAN | 20 plus gateway | /27 | 30 | 9 after 20 endpoints and gateway |
| Voice VLAN | 20 phones plus gateway | /27 | 30 | 9 after 20 phones and gateway |
| Server segment | 2 data servers, voice server and gateway | /28 | 14 | 10 after four addresses |
| Management segment | Core, firewall, router and four switches | /28 | 14 | 6 after eight addresses |

Design decision: Use separate subnets per function and location, maintaining the documented VLAN identifiers 10, 20, 30, 40, 50, 60, 110, 120 and 130. The final address table must allocate exact ranges without overlap.

# 3. Solution B - Bandwidth and capacity engineering

Assume an engineering estimate of 87.2 kbit/s per active G.711 VoIP call including protocol overhead. Twenty simultaneous calls require 20 x 87.2 = 1,744 kbit/s, or 1.744 Mbit/s. Adding 25% design headroom gives 2.18 Mbit/s. If video requires 4 Mbit/s and burst data reaches 60 Mbit/s, total planned load is 66.18 Mbit/s. On a 100 Mbit/s link, utilisation is 66.18%; on a 1 Gbit/s uplink it is 6.618%. This supports Gigabit access/core uplinks and QoS classification rather than relying on a congested 100 Mbit/s bottleneck.

| Traffic class | Estimated load | Priority |
| --- | --- | --- |
| Voice including headroom | 2.18 Mbit/s | Strict/expedited |
| Video conference | 4.00 Mbit/s | High |
| Data burst | 60.00 Mbit/s | Normal |
| Total | 66.18 Mbit/s | Link planning value |

# 4. Solution C - Latency and queueing mathematics

For an M/M/1 approximation, utilisation rho = lambda / mu and mean system time W = 1 / (mu - lambda), valid only when lambda < mu. For 1500-byte packets on a 100 Mbit/s link, mu = 100,000,000 / (1500 x 8) = 8,333 packets/s. At lambda = 5,000 packets/s, rho = 0.60, W = 0.30 ms and average packets in system L = lambda x W = 1.5. At lambda = 7,500 packets/s, rho = 0.90, W = 1.20 ms and L = 9. Once lambda reaches or exceeds 8,333 packets/s, the simple queue is unstable and delay grows without bound.

| Arrival rate lambda | Utilisation rho | Mean W | Mean L | Interpretation |
| --- | --- | --- | --- | --- |
| 5,000 pkt/s | 0.60 | 0.30 ms | 1.5 packets | Healthy with headroom |
| 7,500 pkt/s | 0.90 | 1.20 ms | 9 packets | Congestion risk; protect voice/video |
| 9,000 pkt/s | >1.00 | Unbounded | Unbounded | Unstable; rate-limit, upgrade or redistribute traffic |

A practical one-way latency budget can be expressed as D_total = D_processing + D_queue + D_transmission + D_propagation. A planning example of 2 ms LAN processing + 2 ms firewall/VPN processing + 20 ms queueing + 40 ms WAN path gives 64 ms, below the project voice target of 150 ms. PRTG Ping/Ping Jitter and controlled call tests are required to replace estimates with measured values.

# 5. Solution D - DSP mathematics

For N-point spectral analysis, frequency resolution is delta_f = Fs / N. At Fs = 8,000 Hz and N = 1,024, delta_f = 7.8125 Hz. Signal-to-noise ratio is SNR_dB = 20 log10(A_signal / A_noise) for comparable amplitude measurements; a ratio of 1.0 to 0.1 gives 20 dB. Linear convolution of x = [1, 2, 1] with h = [1, 0.5] produces [1, 2.5, 2, 0.5]. The mathematics supports selection and verification of filters used to reduce unwanted noise or model a channel response.

# 6. Solution E - Reliability and availability

Availability A = MTBF / (MTBF + MTTR). The permitted monthly downtime can also be estimated from (1 - A) x 30 x 24 x 60 minutes. At 99.9% availability, the allowance is approximately 43.8 minutes per 30-day month. At 99.99%, it is approximately 4.38 minutes. The difference demonstrates why fast detection, configuration backups, UPS support and documented recovery procedures are necessary.

| Target | Maximum 30-day downtime | Project implication |
| --- | --- | --- |
| 99.0% | 7 h 12 min | Insufficient for critical lab communication |
| 99.9% | 43.8 min | Reasonable minimum operational target |
| 99.99% | 4.38 min | Requires stronger redundancy and rapid recovery |

# 8. Conclusion

The calculations support the selected Network 3 architecture and provide traceable engineering justification for subnet sizes, Gigabit uplinks, QoS, DSP methods, PRTG monitoring and recovery planning. Final acceptance should compare the calculated expectations with measured evidence and record any variance.

# 9. Sign-off

| Role | Name | Decision | Date |
| --- | --- | --- | --- |
| Prepared by | Leonardo Matias Ciarcia | Completed | 15/03/2026 |
| Engineer reviewer | Tommy | Approved | 15/03/2026 |
| Project sponsor | Dijo | Accepted | 15/03/2026 |