# CONTROL SYSTEMS DESIGN WITH QUEUES

Control Systems Design with Queues Report

Project: Darvycom Brisbane Engineering Laboratory Network and DSP Upgrade
Unit: ICTTEN827
Location: Blocks A and B, Brisbane Engineering Laboratory
Project Manager: Leonardo Matias Ciarcia
Document ID: C6-QUEUE-CONTROL-02
Version: 1.1
Date: 16 March 2026

- Executive summary

This report presents a queue-based control system for the Darvycom Network output interface. The model receives voice, video and general data packets, classifies them into three logical queues and adjusts control actions according to queue occupancy, delay, utilisation and packet drops.

The design uses priority service for voice, weighted service for video and controlled remaining capacity for general data. Four controller states are used: GREEN, AMBER, RED and RECOVERY.

The purpose of the simulation is to evaluate congestion behaviour before QoS settings are implemented on the operational network.

## 2. Control-system objectives

The control system must:

- protect VoIP and video traffic;

- identify increasing congestion;

- defer non-critical traffic during busy periods;

- rate-limit normal data during overload;

- prevent uncontrolled queue growth;

- record packet delay and drops;

- recover gradually after congestion;

- avoid starvation of lower-priority traffic;

- provide information that can be monitored and reported.

## 3. System boundary and assumptions

| Element | Model value |
| --- | --- |
| Input traffic | Voice, video and normal data |
| Output server | One monitored network interface |
| Test capacity | 100 Mbit/s |
| Production uplinks | Gigabit Ethernet |
| Average analytical packet size | 1,500 bytes |
| Queue capacity | 1,000 packets |
| Controller sample period | 1 second |
| Simulation processing | Packet-based or small time-step |
| Main outputs | Occupancy, delay, utilisation, drops and state history |

The 100 Mbit/s link is used for the simulation because congestion can be demonstrated more clearly than on the production Gigabit uplink.

## 4. Queue architecture

| Queue | Traffic class | Service method | Objective |
| --- | --- | --- | --- |
| Q1 | VoIP signalling and RTP | Strict priority with policing | Lowest delay and loss |
| Q2 | Video conferencing | Weighted service after Q1 | Stable video delivery |
| Q3 | General data and backups | Remaining weighted capacity | Protect real-time services |

Strict priority is not unlimited. A policing or service-budget rule is required so that Q1 cannot permanently prevent Q2 and Q3 from receiving service.

## 5. Queue equations

Service rate:

μ = C / (L × 8)

For C = 100,000,000 bit/s and L = 1,500 bytes:

μ = 8,333.33 packets/s

Queue utilisation:

ρ = λ / μ

Queue change:

dq/dt = λ − μ

Stable condition:

λ < μ

Unstable condition:

λ ≥ μ

For the overload test:

λ = 9,000 packets/s
μ = 8,333 packets/s
ρ = 1.08

Because ρ is greater than 1, the queue grows unless the controller applies packet dropping, shaping or rate limiting.

## 6. Controller states

| State | Occupancy | Controller action | Exit condition |
| --- | --- | --- | --- |
| GREEN | Below 50% | Normal scheduling and monitoring | Occupancy reaches 50% |
| AMBER | 50%–75% | Increase real-time protection, warn operator and defer backups | Below 45% or reaches 75% |
| RED | Above 75% | Rate-limit Q3, apply early data drops and preserve Q1/Q2 | Below 65% for five samples |
| RECOVERY | Falling after RED | Gradually restore Q3 and check stability | Below 45% for five samples |

Different entry and exit thresholds provide hysteresis. This reduces rapid switching when queue occupancy remains near one threshold.

## 7. Control logic

- Generate voice, video and data packet-arrival events.

- Classify each packet into Q1, Q2 or Q3.

- Check available queue capacity.

- If capacity is available, enqueue the packet.

- If capacity is exceeded, record a class-specific packet drop.

- When the output link becomes available, service Q1 within its configured budget.

- Service Q2 using its assigned weight.

- Allocate the remaining service capacity to Q3.

- Calculate waiting time and queue delay.

- Every second, calculate occupancy, utilisation, drops and delay.

- Select the correct controller state.

- Record state changes and control actions.

- Export the graphs and final scenario table.

## 8. MATLAB-style pseudocode

Initialise event list, Q1, Q2 and Q3

Set controller state to GREEN

While simulation time is less than duration:

Remove the next event

If event is ARRIVAL:

Classify packet

Add packet to Q1, Q2 or Q3

Record a drop if capacity is exceeded

If event is DEPARTURE:

Select next packet using priority and weights

Record waiting time

Transmit next packet

If event is SAMPLE:

Calculate occupancy

Calculate delay and utilisation

Calculate drops

Apply controller-state rules

Record the state and action

End simulation

Export queue length, delay, drops, utilisation and state history

## 9. Test scenarios

| Scenario | Arrival rate | ρ | Expected behaviour | Controller state |
| --- | --- | --- | --- | --- |
| S1 – Normal | 5,000 pkt/s | 0.60 | Low and stable queue | GREEN |
| S2 – Busy | 7,500 pkt/s | 0.90 | Delay becomes sensitive to traffic variation | GREEN/AMBER |
| S3 – Overload | 9,000 pkt/s | 1.08 | Queue reaches capacity without control | RED |
| S4 – Recovery | 4,000 pkt/s | 0.48 | Backlog drains | RECOVERY then GREEN |

## 10. Representative calculated outputs

| Metric | S1 | S2 | S3 | S4 |
| --- | --- | --- | --- | --- |
| Arrival rate | 5,000 | 7,500 | 9,000 | 4,000 pkt/s |
| Service rate | 8,333 | 8,333 | 8,333 | 8,333 pkt/s |
| Utilisation | 60% | 90% | 108% | 48% |
| Average M/M/1 time | 0.30 ms | 1.20 ms | Unstable | 0.231 ms |
| Packet drops | None expected | Possible during bursts | Required when full | None expected |
| Main action | Monitor | Warn/defer backup | Limit Q3/protect Q1/Q2 | Restore gradually |

These figures are analytical outputs. The final evidence must include screenshots of the executed simulation to verify the calculated behaviour.

## 11. Queue overflow and recovery

During S3:

Queue growth = 9,000 − 8,333
Queue growth = 667 packets/s

Starting from an empty queue:

Time to reach 50% occupancy = 500 / 667
Time = approximately 0.75 seconds

Time to reach 75% occupancy = 750 / 667
Time = approximately 1.12 seconds

Time to reach full capacity = 1,000 / 667
Time = approximately 1.50 seconds

During S4:

Drain rate = 8,333 − 4,000
Drain rate = 4,333 packets/s

Time to drain 1,000 packets:

Time = 1,000 / 4,333
Time = approximately 0.231 seconds

The controller remains in RECOVERY for the configured stability period even if the mathematical queue drains quickly.

## 12. Control actions by traffic class

### Voice

- maintain the highest scheduling priority;

- preserve low delay and jitter;

- police excessive voice traffic;

- alarm if voice performance exceeds its target.

### Video

- allocate a guaranteed weighted share;

- reduce normal data before reducing video;

- monitor delay, jitter and packet loss;

- preserve sufficient throughput for conferencing.

### General data

- use available capacity after voice and video requirements;

- defer backups during AMBER;

- rate-limit large transfers during RED;

- restore bandwidth gradually during RECOVERY.

## 13. Acceptance criteria

| Criterion | Pass condition |
| --- | --- |
| Voice protection | Q1 has the lowest measured waiting time |
| Controlled overload | Queue remains bounded or documented drops occur |
| Recovery | Controller returns to GREEN after the stability period |
| Fairness | Q2 and Q3 continue receiving service |
| Observability | Every state change is timestamped |
| Monitoring | Delay, drops, occupancy and utilisation are exported |
| Stability | Controller does not repeatedly oscillate between states |

## 14. Risks and limitations

| Risk or limitation | Treatment |
| --- | --- |
| Real packets have different sizes | Use traffic-specific distributions in an extended model |
| Arrivals are not perfectly exponential | Test burst and deterministic scenarios |
| Strict priority may cause starvation | Apply policing and minimum service weights |
| Thresholds may cause rapid switching | Use hysteresis and five-sample recovery checks |
| Home monitoring is not production data | Label PRTG evidence as representative |
| Packet Tracer has simulation limitations | Compare with calculations and documented assumptions |

## 15. Conclusion

The queue-based control system provides a repeatable method for evaluating QoS and congestion control before Network solution is placed into operation. The design protects voice and video, controls general data during overload, records measurable outputs and restores capacity gradually after congestion.