DSP design criteria to Darvycom Australia engineering Laboratory network system.

MATLAB PROGRAM:

clear; clc;

close all force;

set(groot,'defaultFigureVisible','on');

rng(7);

%% Darvycom Australia - DSP Application for Engineering Laboratory Network

% Robust version: draws figures explicitly and saves PNG outputs.

fprintf('===============================================================\n');

fprintf('Darvycom Australia - DSP Design and Simulation (Robust Version)\n');

fprintf('===============================================================\n\n');

%% 1) Review DSP criteria applied to signal processing

fs = 8000;                    % Sampling frequency for voice/data analysis

voice_band = [300 3400];      % Typical voice band for VoIP-type signals

snr_target_db = 20;           % Target signal-to-noise ratio

max_delay_ms = 150;           % Real-time communication delay target

interference_freq = 3000;     % Example unwanted component to analyse/filter

fprintf('1) DSP design criteria used in this simulation:\n');

fprintf('   - Sampling frequency: %d Hz\n', fs);

fprintf('   - Voice/baseband operating band: %d Hz to %d Hz\n', voice_band(1), voice_band(2));

fprintf('   - Target SNR for clear communication: %.1f dB\n', snr_target_db);

fprintf('   - Real-time communication delay target: <= %.0f ms\n', max_delay_ms);

fprintf('   - Important DSP goals: noise reduction, stability, bandwidth efficiency, and reliable recovery of digital data\n\n');

%% 2) Perform frequency domain analysis on network signals

T = 0.05;

t = 0:1/fs:T-1/fs;

voice_component = 0.8*sin(2*pi*500*t) + 0.45*sin(2*pi*1200*t);

interference_component = 0.35*sin(2*pi*interference_freq*t);

noise_component = 0.20*randn(size(t));

network_signal = voice_component + interference_component + noise_component;

Nfft = 2^nextpow2(length(network_signal));

X = fft(network_signal, Nfft);

f = (0:Nfft/2-1)*(fs/Nfft);

magX = abs(X(1:Nfft/2))/length(network_signal);

fig1 = figure('Name','Darvycom Question 2 - Frequency Domain Analysis', ...

'NumberTitle','off','Color','w');

subplot(2,1,1);

plot(t, network_signal, 'LineWidth', 1.2);

xlabel('Time (s)'); ylabel('Amplitude');

title('Simulated Darvycom network signal in time domain');

grid on; axis tight;

subplot(2,1,2);

plot(f, magX, 'LineWidth', 1.2);

xlabel('Frequency (Hz)'); ylabel('Magnitude');

title('Frequency domain analysis using FFT');

grid on; axis tight;

xlim([0 fs/2]);

drawnow;

try

exportgraphics(fig1, 'darvycom_q2_frequency_analysis.png', 'Resolution', 200);

catch

saveas(fig1, 'darvycom_q2_frequency_analysis.png');

end

[~, idx500] = min(abs(f - 500));

[~, idx1200] = min(abs(f - 1200));

[~, idx3000] = min(abs(f - interference_freq));

fprintf('2) Frequency-domain analysis results:\n');

fprintf('   - Main useful component near 500 Hz, magnitude %.4f\n', magX(idx500));

fprintf('   - Main useful component near 1200 Hz, magnitude %.4f\n', magX(idx1200));

fprintf('   - Unwanted interference near %d Hz, magnitude %.4f\n\n', interference_freq, magX(idx3000));

%% 3) Apply digital processing techniques to DSP baseband communication signals

numBits = 20;

bitRate = 100;

samplesPerBit = 20;

fs_bb = bitRate * samplesPerBit;

bits = randi([0 1], 1, numBits);

symbols = 2*bits - 1;

tx_baseband = repelem(symbols, samplesPerBit);

tbb = (0:length(tx_baseband)-1)/fs_bb;

channel_noise = 0.75*randn(size(tx_baseband));

hum_noise = 0.30*sin(2*pi*50*tbb);

rx_noisy = tx_baseband + channel_noise + hum_noise;

h_lp = ones(1, samplesPerBit)/samplesPerBit;

rx_filtered = conv(rx_noisy, h_lp, 'same');

sample_points = round(samplesPerBit/2):samplesPerBit:length(rx_filtered);

recovered_before = rx_noisy(sample_points) > 0;

recovered_after = rx_filtered(sample_points) > 0;

ber_before = sum(recovered_before ~= bits)/numBits;

ber_after = sum(recovered_after ~= bits)/numBits;

fig2 = figure('Name','Darvycom Question 3 - Baseband DSP Processing', ...

'NumberTitle','off','Color','w');

subplot(3,1,1);

plot(tbb, tx_baseband, 'LineWidth', 1.2);

xlabel('Time (s)'); ylabel('Amplitude');

title('Transmitted baseband digital signal');

ylim([-2 2]); grid on; axis tight;

subplot(3,1,2);

plot(tbb, rx_noisy, 'LineWidth', 1.0);

xlabel('Time (s)'); ylabel('Amplitude');

title('Received signal with noise/interference');

ylim([-3 3]); grid on; axis tight;

subplot(3,1,3);

plot(tbb, rx_filtered, 'LineWidth', 1.1); hold on;

stem(tbb(sample_points), rx_filtered(sample_points), 'filled');

xlabel('Time (s)'); ylabel('Amplitude');

title('Filtered signal with symbol sampling points');

ylim([-2 2]); grid on; axis tight;

hold off;

drawnow;

try

exportgraphics(fig2, 'darvycom_q3_baseband_processing.png', 'Resolution', 200);

catch

saveas(fig2, 'darvycom_q3_baseband_processing.png');

end

fprintf('3) Baseband DSP processing results:\n');

fprintf('   - Original bits:             '); fprintf('%d ', bits); fprintf('\n');

fprintf('   - Recovered bits before DSP: '); fprintf('%d ', recovered_before); fprintf('\n');

fprintf('   - Recovered bits after DSP:  '); fprintf('%d ', recovered_after); fprintf('\n');

fprintf('   - BER before filtering = %.3f\n', ber_before);

fprintf('   - BER after filtering  = %.3f\n\n', ber_after);

%% 4) Generate comb filter output using convolution theorem and autocorrelation

T2 = 0.04;

t2 = 0:1/fs:T2-1/fs;

desired_signal = 0.6*sin(2*pi*400*t2);

periodic_interference = 0.7*sin(2*pi*1000*t2);

x_comb = desired_signal + periodic_interference + 0.10*randn(size(t2));

alpha = 0.9;

N = round(fs/1000);

h_comb = [1 zeros(1, N-1) -alpha];

y_conv = conv(x_comb, h_comb, 'full');

L = length(x_comb) + length(h_comb) - 1;

y_fft = real(ifft(fft(x_comb, L) .* fft(h_comb, L)));

conv_error = max(abs(y_conv - y_fft));

auto_x = conv(x_comb, fliplr(x_comb));

lags = -(length(x_comb)-1):(length(x_comb)-1);

H_comb = fft(h_comb, 1024);

fH = (0:511)*(fs/1024);

magH = abs(H_comb(1:512));

fig3 = figure('Name','Darvycom Question 4 - Comb Filter, Convolution and Autocorrelation', ...

'NumberTitle','off','Color','w');

subplot(2,2,1);

plot(t2, x_comb, 'LineWidth', 1.0);

xlabel('Time (s)'); ylabel('Amplitude');

title('Input signal with periodic interference');

grid on; axis tight;

subplot(2,2,2);

plot((0:length(y_conv)-1)/fs, y_conv, 'LineWidth', 1.0);

xlabel('Time (s)'); ylabel('Amplitude');

title('Comb filter output from convolution');

grid on; axis tight;

subplot(2,2,3);

plot(lags, auto_x, 'LineWidth', 1.0);

xlabel('Lag'); ylabel('Autocorrelation');

title('Autocorrelation of input signal');

grid on; axis tight;

subplot(2,2,4);

plot(fH, magH, 'LineWidth', 1.1);

xlabel('Frequency (Hz)'); ylabel('|H(f)|');

title('Comb filter magnitude response');

grid on; axis tight;

xlim([0 fs/2]);

drawnow;

try

exportgraphics(fig3, 'darvycom_q4_comb_filter.png', 'Resolution', 200);

catch

saveas(fig3, 'darvycom_q4_comb_filter.png');

end

fprintf('4) Comb filter results:\n');

fprintf('   - Comb filter equation: h[n] = delta[n] - alpha*delta[n-N]\n');

fprintf('   - Output equation:      y[n] = x[n] * h[n] = x[n] - alpha*x[n-N]\n');

fprintf('   - Convolution theorem check max error = %.12f\n', conv_error);

fprintf('   - Delay used N = %d samples\n', N);

fprintf('   - Scale factor alpha = %.2f\n\n', alpha);

fprintf('Simulation complete.\n');

fprintf('PNG files saved in the current MATLAB folder:\n');

fprintf('   - darvycom_q2_frequency_analysis.png\n');

fprintf('   - darvycom_q3_baseband_processing.png\n');

fprintf('   - darvycom_q4_comb_filter.png\n');

EXPLANATION:

reviews the DSP design criteria: In this part, the code defines the main parameters used in the simulation, such as sampling frequency, voice frequency range, target signal-to-noise ratio, delay limit and interference frequency. These values represent the quality and performance requirements for Darvycom’s upgraded network.

performs frequency domain analysis: The code creates a network signal made of useful communication frequencies, interference and random noise. Then it uses FFT to convert the signal from the time domain to the frequency domain. The output graph shows the main useful frequencies and also the interference peak, helping to identify what should be filtered.

applies DSP techniques to a baseband digital signal: The code generates random bits, converts them into a digital waveform, adds noise and interference, and then applies a low-pass filter. The output shows the transmitted signal, the noisy received signal and the filtered signal. It also calculates the bit error rate before and after filtering, showing that DSP improves signal recovery.

generates the output of a comb filter: The code creates a signal with periodic interference, applies the comb filter using convolution, and checks the same result using the convolution theorem. It also calculates autocorrelation to show repeated patterns in the signal. The output graphs show the input signal, the filtered output, the autocorrelation and the comb filter response.

OUTPUT: