Future Network Traffic Trends and Capacity Planning

Project: Cluster 6 – Darvycom Australia
Unit: ICTTEN615 – Manage Network Traffic
Prepared by: Leonardo Matias Ciarcia
Date: 4/02/2026
Version: 1.0

Purpose

This section forecasts the future network traffic requirements of Darvycom’s two Engineering Laboratories. The forecast is based on the current network design, historical PRTG monitoring results, existing users and devices, expected application growth and future business requirements.

Current network baseline

The final Darvycom network solution supports approximately:

- 20 engineering-lab users.

- 20 computers.

- 20 IP phones.

- Two wireless access points.

- One wireless laptop.

- One video-conference device.

- Three shared service devices.

- Four access switches.

- One Layer 3 core switch.

- One firewall.

- One VPN edge router.

- One ISP router.

The environment therefore contains approximately 55 network devices and endpoints. PRTG monitoring showed normal traffic patterns with temporary peaks caused by video conferencing, engineering file transfers, wireless activity and connectivity testing.

[Insert or refer to P2-E6 here – PRTG historical traffic graph. If this screenshot was already inserted earlier, write: “Refer to P2-E6 previously provided.”]

Expected future network use

Darvycom is expected to increase its use of digital engineering systems and network-based services. Future network use may include:

- Additional engineering workstations.

- DSP and simulation applications.

- Larger engineering and technical files.

- Cloud storage and collaboration.

- More simultaneous IP telephone calls.

- Additional video-conference sessions.

- More wireless laptops and mobile devices.

- Remote access through the VPN.

- Increased server access.

- Automated backups and software updates.

- Continuous PRTG and security monitoring.

Future traffic forecast

For planning purposes, a peak traffic baseline of 250 Mbps and an estimated annual traffic-growth rate of 25% were used.

The forecast was calculated using:

Future Traffic = Current Peak Traffic × (1 + Annual Growth Rate)ⁿ

Year 1:

250 Mbps × 1.25 = 312.5 Mbps

Year 2:

250 Mbps × 1.25² = 390.6 Mbps

Year 3:

250 Mbps × 1.25³ = 488.3 Mbps

| Forecast period | Expected users | Estimated devices | Forecast peak traffic |
| --- | --- | --- | --- |
| Current baseline | 20 | Approximately 55 | 250 Mbps |
| Year 1 | 24 | Approximately 60 | 312.5 Mbps |
| Year 2 | 28 | Approximately 68 | 390.6 Mbps |
| Year 3 | 32 | Approximately 76 | 488.3 Mbps |

[Insert P2-D1 here – screenshot of this forecast table. You may also create a simple column chart showing 250, 312.5, 390.6 and 488.3 Mbps.]

Expected traffic by application

| Application or service | Expected traffic behaviour | Potential impact |
| --- | --- | --- |
| Engineering and DSP applications | Large technical files and temporary processing peaks | Increased traffic on laboratory, server and core links |
| Video conferencing | Continuous real-time audio and video | Sensitive to delay, jitter and packet loss |
| IP telephony | Multiple simultaneous voice calls | Requires QoS and stable low-latency connectivity |
| Wireless devices | Increasing connections from laptops and mobile devices | Greater access-point utilisation and possible wireless congestion |
| Cloud and internet services | Email, research, collaboration and cloud storage | Increased firewall, VPN and internet-link utilisation |
| Shared servers | File, data, voice and application services | Increased server-link, processing and storage demand |
| Backups and software updates | Large scheduled transfers | Temporary congestion if performed during peak periods |
| Monitoring and security | Continuous PRTG, logging and security traffic | Small but continuous management traffic |

Potential impact on the network

The forecast indicates that peak traffic could approach approximately 490 Mbps within three years. Gigabit uplinks should provide sufficient capacity if traffic remains properly managed and monitored. However, any remaining 100 Mbps connection could become a bottleneck.

The greatest capacity pressure is expected on:

- The ISP and internet-facing connection.

- The connection between the firewall and Core-L3 switch.

- The Engineering Lab access-switch uplinks.

- Shared server connections.

- Wireless access points.

- Voice and video services during peak periods.

If several users perform large file transfers while video conferences and IP calls are active, latency and packet loss could increase. This could result in slow application response, interrupted video and reduced voice quality.

Capacity-planning thresholds

| Status | Link utilisation | Required action |
| --- | --- | --- |
| Normal | Below 70% | Continue normal monitoring |
| Warning | 70–84% | Investigate traffic source and review available capacity |
| Critical | 85% or higher | Apply traffic controls and prepare a capacity upgrade |
| Severe | 95% or higher | Initiate congestion recovery and escalation |

Recommendations

Based on the traffic forecast, the following actions are recommended:

- Maintain Gigabit uplinks between the access switches, Core-L3 switch and shared services.

- Maintain at least 20–30% spare capacity on critical network links.

- Use PRTG to continuously monitor utilisation, latency, packet loss, availability, errors and discards.

- Begin a formal capacity review when link utilisation remains above 70%.

- Implement corrective or upgrade action when utilisation regularly exceeds 85%.

- Configure QoS to prioritise IP voice and video-conference traffic.

- Schedule backups, software updates and large file transfers outside peak periods.

- Review wireless performance as the number of users and devices increases.

- Consider link aggregation or redundant links for important core and server connections.

- Review ISP bandwidth, server capacity and monitoring licences before the Year 3 forecast is reached.

- Update the traffic forecast every six months using new PRTG historical data.

- Use the formal change and budget approval processes before implementing major capacity upgrades.

Conclusion

The analysis indicates that Darvycom’s network traffic may increase significantly due to additional users, engineering applications, wireless devices, IP telephony, video conferencing and shared services. The final network solution provides a suitable foundation for this growth, but continued PRTG monitoring, QoS, scheduled traffic management and regular capacity reviews will be required.

Implementing the recommendations will help Darvycom maintain reliable network performance and prevent future congestion across Engineering Lab 1, Engineering Lab 2 and the shared network services.