# IP ADDRESS TABLE (Current x Requested)

Project: Darvycom Brisbane Engineering Laboratory Network & DSP Upgrade
Project Manager:  Leonardo Matias Ciarcia
Date: 02/01/2026

- PR = Private IP address

- PU = Simulated public IP address

- “Class” follows the traditional IP address class shown in the example. The actual network design uses CIDR subnet masks.

- Switch trunk ports and Layer 2 access ports do not receive IP addresses. Switches are managed through their management VLAN interface.

- Panasonic PBXs, analogue phones, headsets and unmanaged UPS devices do not require IP addresses.

# IP Address Table – Network 1: Current Network

## Network 1 gateway information

| Internal network | Default gateway | Subnet mask |
| --- | --- | --- |
| Block A network | 192.168.10.1 | 255.255.255.0 /24 |
| Block B network | 192.168.20.1 | 255.255.255.0 /24 |
| Block A WAN link | 172.16.0.1 | 255.255.255.252 /30 |
| Block B WAN link | 172.16.0.5 | 255.255.255.252 /30 |
| Simulated public test network | 203.0.114.1 | 255.255.255.0 /24 |

## Network 1 IP schema

| Device/host/node | IP address or IPv4 range | Public or Private | IP type | Subnet | Class |
| --- | --- | --- | --- | --- | --- |
| ISP-CURRENT – G0/0 | 172.16.0.1 | PR | Static | 255.255.255.252 /30 | B |
| R-A-CURRENT – G0/1 WAN | 172.16.0.2 | PR | Static | 255.255.255.252 /30 | B |
| ISP-CURRENT – G0/1 | 172.16.0.5 | PR | Static | 255.255.255.252 /30 | B |
| R-B-CURRENT – G0/1 WAN | 172.16.0.6 | PR | Static | 255.255.255.252 /30 | B |
| ISP-CURRENT – G0/2 | 203.0.114.1 | PU – simulated public | Static | 255.255.255.0 /24 | C |
| INTERNET-TEST | 203.0.114.10 | PU – simulated public | Static | 255.255.255.0 /24 | C |
| R-A-CURRENT – G0/0 LAN | 192.168.10.1 | PR | Static | 255.255.255.0 /24 | C |
| SW-A1-CURRENT – VLAN 1 | 192.168.10.2 | PR | Static | 255.255.255.0 /24 | C |
| SW-A2-CURRENT – VLAN 1 | 192.168.10.3 | PR | Static | 255.255.255.0 /24 | C |
| DATA-SRV-A | 192.168.10.10 | PR | Static | 255.255.255.0 /24 | C |
| PC-A01 to PC-A08 | 192.168.10.101–192.168.10.108 | PR | Static | 255.255.255.0 /24 | C |
| R-B-CURRENT – G0/0 LAN | 192.168.20.1 | PR | Static | 255.255.255.0 /24 | C |
| SW-B1-CURRENT – VLAN 1 | 192.168.20.2 | PR | Static | 255.255.255.0 /24 | C |
| SW-B2-CURRENT – VLAN 1 | 192.168.20.3 | PR | Static | 255.255.255.0 /24 | C |
| DATA-SRV-B | 192.168.20.10 | PR | Static | 255.255.255.0 /24 | C |
| PC-B01 to PC-B08 | 192.168.20.101–192.168.20.108 | PR | Static | 255.255.255.0 /24 | C |

### Network 1 endpoint gateways

| Devices | Default gateway |
| --- | --- |
| SW-A1, SW-A2, DATA-SRV-A and PC-A01–PC-A08 | 192.168.10.1 |
| SW-B1, SW-B2, DATA-SRV-B and PC-B01–PC-B08 | 192.168.20.1 |
| INTERNET-TEST | 203.0.114.1 |
| R-A-CURRENT default route | 172.16.0.1 |
| R-B-CURRENT default route | 172.16.0.5 |

### Network 1 devices without IP addresses

| Device | IP requirement |
| --- | --- |
| PANASONIC-PBX-A | Not applicable – legacy analogue PBX |
| PANASONIC-PBX-B | Not applicable – legacy analogue PBX |
| Analogue telephones | Not applicable |
| Unmanaged UPS devices | Not applicable |

# IP Address Table – Network 3: Final Selected Network

## VLAN and default gateway schema

| VLAN | Network purpose | Network address | Default gateway | Subnet mask |
| --- | --- | --- | --- | --- |
| 10 | DATA-A | 10.10.10.0 | 10.10.10.1 | 255.255.255.0 /24 |
| 20 | VOICE-A | 10.10.20.0 | 10.10.20.1 | 255.255.255.0 /24 |
| 30 | WLAN-A | 10.10.30.0 | 10.10.30.1 | 255.255.255.0 /24 |
| 40 | SERVERS | 10.10.40.0 | 10.10.40.1 | 255.255.255.0 /24 |
| 50 | MANAGEMENT | 10.10.50.0 | 10.10.50.1 | 255.255.255.0 /24 |
| 110 | DATA-B | 10.20.10.0 | 10.20.10.1 | 255.255.255.0 /24 |
| 120 | VOICE-B | 10.20.20.0 | 10.20.20.1 | 255.255.255.0 /24 |
| 130 | WLAN-B | 10.20.30.0 | 10.20.30.1 | 255.255.255.0 /24 |
| N/A | Core-to-firewall transit | 10.255.0.0 | Point-to-point link | 255.255.255.252 /30 |
| N/A | Firewall-to-VPN Edge | 198.51.100.0 | Point-to-point link | 255.255.255.252 /30 |
| N/A | VPN Edge-to-ISP | 203.0.113.0 | Point-to-point link | 255.255.255.252 /30 |
| N/A | Simulated Internet test LAN | 203.0.114.0 | 203.0.114.1 | 255.255.255.0 /24 |

## Core, security and WAN devices

| Device/host/node | IP address or IPv4 range | Public or Private | IP type | Subnet | Class |
| --- | --- | --- | --- | --- | --- |
| CORE-L3 – VLAN 10 SVI | 10.10.10.1 | PR | Static | 255.255.255.0 /24 | A |
| CORE-L3 – VLAN 20 SVI | 10.10.20.1 | PR | Static | 255.255.255.0 /24 | A |
| CORE-L3 – VLAN 30 SVI | 10.10.30.1 | PR | Static | 255.255.255.0 /24 | A |
| CORE-L3 – VLAN 40 SVI | 10.10.40.1 | PR | Static | 255.255.255.0 /24 | A |
| CORE-L3 – VLAN 50 SVI | 10.10.50.1 | PR | Static | 255.255.255.0 /24 | A |
| CORE-L3 – VLAN 110 SVI | 10.20.10.1 | PR | Static | 255.255.255.0 /24 | A |
| CORE-L3 – VLAN 120 SVI | 10.20.20.1 | PR | Static | 255.255.255.0 /24 | A |
| CORE-L3 – VLAN 130 SVI | 10.20.30.1 | PR | Static | 255.255.255.0 /24 | A |
| CORE-L3 – G0/1 to ASA | 10.255.0.2 | PR | Static | 255.255.255.252 /30 | A |
| ASA Firewall – Inside | 10.255.0.1 | PR | Static | 255.255.255.252 /30 | A |
| ASA Firewall – Outside | 198.51.100.2 | PU – simulated public | Static | 255.255.255.252 /30 | C |
| VPN Edge Router – Inside | 198.51.100.1 | PU – simulated public | Static | 255.255.255.252 /30 | C |
| VPN Edge Router – Outside | 203.0.113.2 | PU – simulated public | Static | 255.255.255.252 /30 | C |
| ISP Router – G0/0 | 203.0.113.1 | PU – simulated public | Static | 255.255.255.252 /30 | C |
| ISP Router – G0/1 | 203.0.114.1 | PU – simulated public | Static | 255.255.255.0 /24 | C |
| INTERNET-TEST | 203.0.114.10 | PU – simulated public | Static | 255.255.255.0 /24 | C |

## Management VLAN devices

All devices in this table use default gateway 10.10.50.1.

| Device/host/node | IP address or IPv4 range | Public or Private | IP type | Subnet | Class |
| --- | --- | --- | --- | --- | --- |
| DIST-A | 10.10.50.3 | PR | Static | 255.255.255.0 /24 | A |
| DIST-B | 10.10.50.4 | PR | Static | 255.255.255.0 /24 | A |
| ACCESS-A1 | 10.10.50.11 | PR | Static | 255.255.255.0 /24 | A |
| ACCESS-A2 | 10.10.50.12 | PR | Static | 255.255.255.0 /24 | A |
| ACCESS-B1 | 10.10.50.21 | PR | Static | 255.255.255.0 /24 | A |
| ACCESS-B2 | 10.10.50.22 | PR | Static | 255.255.255.0 /24 | A |
| WLC-A management interface | 10.10.50.31 | PR | Static | 255.255.255.0 /24 | A |
| WLC-B management interface | 10.10.50.32 | PR | Static | 255.255.255.0 /24 | A |
| AP-A1 | 10.10.50.41 | PR | Static | 255.255.255.0 /24 | A |
| AP-A2 | 10.10.50.42 | PR | Static | 255.255.255.0 /24 | A |
| AP-B1 | 10.10.50.43 | PR | Static | 255.255.255.0 /24 | A |
| AP-B2 | 10.10.50.44 | PR | Static | 255.255.255.0 /24 | A |

## Server VLAN devices

All devices in this table use default gateway 10.10.40.1.

| Device/host/node | IP address or IPv4 range | Public or Private | IP type | Subnet | Class |
| --- | --- | --- | --- | --- | --- |
| DATA-SRV1 | 10.10.40.10 | PR | Static | 255.255.255.0 /24 | A |
| DATA-SRV2-PRTG | 10.10.40.11 | PR | Static | 255.255.255.0 /24 | A |
| VOICE-SRV-IP-PBX | 10.10.40.20 | PR | Static | 255.255.255.0 /24 | A |
| NAS-STORAGE | 10.10.40.30 | PR | Static | 255.255.255.0 /24 | A |

## Block A endpoint devices

| Device/host/node | IP address or IPv4 range | Public or Private | IP type | Subnet | Class |
| --- | --- | --- | --- | --- | --- |
| PC-A01 to PC-A10 | 10.10.10.11–10.10.10.20 | PR | Static | 255.255.255.0 /24 | A |
| VIDEO-CONF | 10.10.10.30 | PR | Static | 255.255.255.0 /24 | A |
| PHONE-A01 to PHONE-A10 | 10.10.20.101–10.10.20.110 | PR | DHCP/Dynamic | 255.255.255.0 /24 | A |
| WLC-A WLAN-A interface | 10.10.30.2 | PR | Static | 255.255.255.0 /24 | A |
| WLAN-A wireless clients | 10.10.30.100–10.10.30.199 | PR | DHCP/Dynamic | 255.255.255.0 /24 | A |

Block A gateways:

- PCs and video conference system: 10.10.10.1

- IP phones: 10.10.20.1

- WLAN-A devices: 10.10.30.1

## Block B endpoint devices

| Device/host/node | IP address or IPv4 range | Public or Private | IP type | Subnet | Class |
| --- | --- | --- | --- | --- | --- |
| PC-B01 to PC-B10 | 10.20.10.11–10.20.10.20 | PR | Static | 255.255.255.0 /24 | A |
| PHONE-B01 to PHONE-B10 | 10.20.20.101–10.20.20.110 | PR | DHCP/Dynamic | 255.255.255.0 /24 | A |
| WLC-B WLAN-B interface | 10.20.30.2 | PR | Static | 255.255.255.0 /24 | A |
| WLAN-B wireless clients | 10.20.30.100–10.20.30.199 | PR | DHCP/Dynamic | 255.255.255.0 /24 | A |

Block B gateways:

- PCs: 10.20.10.1

- IP phones: 10.20.20.1

- WLAN-B devices: 10.20.30.1

## Network 3 devices and interfaces without IP addresses

| Device or interface | IP requirement |
| --- | --- |
| Layer 2 access switchports | No IP address |
| Trunk links between switches | No IP address |
| IP phone headsets | No separate IP address |
| Patch panels, racks and cabling | No IP address |
| Unmanaged UPS devices | No IP address |
| PCs connected through IP phone switch ports | PC retains its own DATA VLAN address |