Network Traffic Management Report

Evaluation of Existing Network and Solution

Project: Darvycom Brisbane Engineering Laboratory Network & DSP Upgrade

Location: Blocks A and B, Brisbane Engineering Laboratory

Project Manager: Leonardo Matias Ciarcia

Document ID: C6-NET-TM-01

Version: 1.0

Prepared: 02 March 2026

Baseline: 24 weeks / AUD 184,800

# 1. Executive summary

This report evaluates traffic management across Darvycom Australia's two documented network systems: the existing Network 1 and the selected Network 3. Network 1 is a small two-block environment with limited segmentation and no integrated monitoring. Network 3 introduces a VPN edge, firewall, Layer 3 core, four managed PoE access switches, VLANs, QoS and PRTG monitoring. The design review and completed simulations show that Network 3 provides materially better visibility, isolation, scalability and control of voice, video and data traffic.

Conclusion: Network 3 is the preferred production design. Its VLAN and QoS controls reduce broadcast scope and protect real-time services, while PRTG provides the evidence needed to manage availability, latency, packet loss and interface utilisation.

# 2. Systems evaluated

| Feature | Network 1 - existing | Network 3 - selected |
| --- | --- | --- |
| Topology | Two blocks, separate routers and access switches | Internet/ISP -> VPN router -> firewall -> Layer 3 core -> four PoE access switches |
| Endpoints | 16 PCs and 8 analogue phones | 20 PCs, 20 IP phones, 40 headsets, video endpoint and servers |
| Voice | Legacy Panasonic PBX; separate analogue service | Cisco 2911 CME/IP-PBX and voice VLANs |
| Segmentation | Block A 192.168.10.0/24; Block B 192.168.20.0/24; limited policy separation | VLANs 10, 20, 30, 40, 50, 60, 110, 120 and 130 with inter-VLAN routing and policy boundaries |
| Monitoring | No central performance platform documented | PRTG sensors for ping, interface traffic, system health and alarms |
| Traffic control | Best-effort forwarding | QoS classification, priority for voice/video, managed uplinks and threshold-based alerting |

# 3. Traffic-management method

The assessment uses five management dimensions: reachability, utilisation, delay/packet loss, segmentation and operational response. Packet Tracer validates the logical controls. PRTG provides representative operational evidence from a home-lab router and Windows monitoring host; this evidence demonstrates the monitoring method but must not be described as a measurement taken on Darvycom production equipment.

| Metric | Collection method | Project interpretation | Suggested threshold |
| --- | --- | --- | --- |
| Availability | PRTG Ping sensor and device status | Confirms that routers, servers and key endpoints respond | Warning after 2 failed intervals; down after 3 |
| Latency | Ping/Ping Jitter sensor | Identifies delay changes affecting calls and video | Investigate sustained >100 ms; voice target <150 ms one way |
| Packet loss | Ping and traffic sensors | Indicates congestion, interference or link faults | Investigate >1%; urgent for sustained >3% |
| Interface utilisation | SNMP Traffic 64-bit sensor | Shows inbound/outbound load and congestion risk | Warn at 70%; critical at 85% |
| Errors/discards | SNMP interface channels | Detects cabling, duplex or congestion faults | Any rising trend requires investigation |

# 4. Test scenarios and results

| ID | Scenario | Network 1 expectation | Network 3 result/status | Evidence |
| --- | --- | --- | --- | --- |
| TM-01 | PC-to-gateway and cross-VLAN connectivity | Basic reachability; broad trust between services | Packet Tracer connectivity succeeded after correcting VLAN, trunk, gateway and interface settings | Packet Tracer screenshots and final ping output |
| TM-02 | Voice and data coexistence | Analogue voice is outside IP QoS; no unified view | Voice VLAN and QoS design protect Cisco CME calls while data remains available | Phone registration/call screenshots; QoS configuration |
| TM-03 | Video-to-server traffic | No integrated video service | Video endpoint 10.10.40.10 reaches Server1 10.10.60.10 through controlled routing | Successful ping/service test screenshot |
| TM-04 | Router availability monitoring | Manual fault discovery | PRTG Ping sensor records response time, uptime and packet loss | Embedded representative PRTG evidence |
| TM-05 | Interface bandwidth monitoring | No central traffic graph | PRTG SNMP traffic sensor displays inbound, outbound and total traffic | Embedded representative PRTG evidence |
| TM-06 | Monitoring-server capacity | Not available | PRTG System Health sensor records CPU and available memory | Embedded representative PRTG evidence |

# 5. Comparative evaluation

| Criterion | Network 1 | Network 3 | Evaluation |
| --- | --- | --- | --- |
| Broadcast control | Large per-block subnets | Function-based VLANs | Network 3 reduces unnecessary broadcast exposure |
| Real-time priority | No IP voice policy | QoS for voice/video | Network 3 supports predictable real-time performance |
| Fault isolation | Manual and device-by-device | PRTG alarms plus segmented paths | Network 3 shortens detection and diagnosis |
| Security | Limited internal boundaries | Firewall, VPN, VLANs and ACL-ready design | Network 3 supports least privilege and controlled remote access |
| Scalability | Little documented headroom | Managed access/core hierarchy | Network 3 can expand without redesigning the whole network |

# 6. Management recommendations

Retain Network 3 as the approved baseline and document every VLAN, trunk, gateway and QoS change.

Create PRTG device groups for the VPN router, firewall, core, four access switches, two data servers, Cisco CME voice server, access points and video endpoint.

Apply 60-second scanning to critical reachability sensors and a longer interval to lower-priority capacity sensors.

Configure warning and error thresholds for utilisation, latency, packet loss, interface errors and sensor health.

Run a controlled busy-network test, save before/during/after screenshots and record the time, device and expected result.

Review PRTG trends weekly during stabilisation and monthly after operational acceptance.

# 7. Evidence register

| Evidence item | What it proves | Status / action |
| --- | --- | --- |
| PRTG Ping sensor screen | Reachability, response time, packet loss and uptime collection | Available below; representative home-lab method |
| PRTG SNMP traffic graph | Inbound/outbound traffic measurement on a monitored router interface | Available below; representative home-lab method |
| PRTG System Health graph | Monitoring host CPU and memory capacity | Available below; representative home-lab method |
| Packet Tracer final pings | Network 3 routing and VLAN corrections succeeded | Attach final successful screenshots |
| VoIP registration and call | Cisco CME and voice VLAN operation | Attach phone registration and active-call screenshots |
| PRTG outage and recovery | Alarm transition and recovery timing | Perform controlled Wi-Fi/router outage and attach before/down/recovery screens |

# Representative PRTG evidence

production measurements.

PRTG Ping sensor showing approximately 1 ms response time, 0% packet loss and 100% uptime in the representative router test.

PRTG SNMP Traffic 64-bit graph showing traffic-in, traffic-out, total traffic, uptime and coverage for a monitored router interface.

PRTG System Health graph showing monitoring-host health, CPU load and available memory channels.

# Sign-off

| Role | Name | Decision | Date |
| --- | --- | --- | --- |
| Prepared by | Leonardo Matias Ciarcia | Completed | 02/03/2026 |
| Technical reviewer | Tommy | Approved | 02/03/2026 |
| Project sponsor | Dijo | Accepted | 02/03/2026 |