# PROJECT SPECIFICATIONS AND ATTRIBUTES DOCUMENT

Project: Darvycom Brisbane Engineering Laboratory Network & DSP Upgrade
Project Manager:  Leonardo Matias Ciarcia
Date: 20/01/2026

## Project Overview

Darvycom Australia currently operates a basic data network and a copper-based telephone system across two engineering laboratory blocks in Brisbane. The existing network has limited capacity and supports 16 computers and eight analogue telephones.

The project will replace or upgrade the existing infrastructure to support 20 computers, 20 IP phones, 40 headsets, two data servers, one IP-PBX voice server, one video-conferencing system, managed network switches, secure VPN connectivity and improved network monitoring.

The selected solution uses VLAN segmentation, Quality of Service, managed switching, Voice over IP, network security controls and PRTG Network Monitor. The upgrade is intended to provide sufficient performance for voice, data, video-conferencing and Digital Signal Processing activities.

## Existing Network Specifications and Attributes

| Attribute | Existing network specification |
| --- | --- |
| Network type | Basic Ethernet data network with a separate copper-based telephone system |
| Site arrangement | Two engineering laboratory blocks |
| Network architecture | Each laboratory block has local switches and an internet router |
| Existing computers | 16 computers: PCA1–PCA8 and PCB1–PCB8 |
| Existing telephones | Eight analogue telephones |
| Existing routers | Two internet routers: R-A-Current and R-B-Current |
| Existing switches | Four switches: Sw-a1, Switch2A, Switch1B and Switch2B |
| Voice technology | Traditional analogue telephony over copper cabling |
| Data technology | Basic wired Ethernet connectivity |
| Network segmentation | VLAN-based separation is not documented in the existing network |
| Traffic prioritisation | QoS for voice and video is not documented |
| Remote access | Secure VPN capability is not documented |
| Monitoring | No central PRTG monitoring system is documented in the existing environment |
| Security | Existing security controls are limited and require improvement |
| Capacity | Insufficient for the required 20 computers, 20 IP phones, servers, headsets and video-conferencing services |
| Scalability | Limited capacity for additional users, devices and network services |
| Main limitation | The existing network cannot adequately support the planned integrated voice, data, video and DSP services |

## 4. Existing Hardware Resources

The price, cost and supplier fields are marked as existing assets because the original purchase information was not provided. This avoids using invented financial information.

| Item Type | Vendor | Specifications and Attributes | QTY | Price | Cost | Timeframe | Supplier | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internet routers | Cisco; exact model not recorded | Existing routers identified as R-A-Current and R-B-Current. Provide routing between each laboratory network and the external network or internet connection. VPN capability is not documented. | 2 | Existing asset | No new cost | Already installed | Existing Darvycom asset | One associated with each laboratory block |
| Ethernet switches | Cisco; exact model not recorded | Existing access switches identified as Sw-a1, Switch2A, Switch1B and Switch2B. Provide basic wired connectivity for current computers and network devices. Managed VLAN and PoE capabilities require verification. | 4 | Existing asset | No new cost | Already installed | Existing Darvycom asset | Two switches per laboratory block |
| Desktop computers – Block A | Vendor and model not recorded | Existing workstations identified as PCA1 to PCA8 and connected to the Block A data network. | 8 | Existing asset | No new cost | Already installed | Existing Darvycom asset | Laboratory Block A |
| Desktop computers – Block B | Vendor and model not recorded | Existing workstations identified as PCB1 to PCB8 and connected to the Block B data network. | 8 | Existing asset | No new cost | Already installed | Existing Darvycom asset | Laboratory Block B |
| Analogue telephones | Vendor and model not recorded | Existing copper-based telephone handsets. They do not provide the required IP telephony, voice VLAN or network-based call management functions. | 8 | Existing asset | No new cost | Already installed | Existing Darvycom asset | Across both laboratory blocks |
| Analogue telephone system | Vendor and model not recorded | Existing legacy telephone service that supports the eight analogue telephones. Exact PBX or controller model must be confirmed through a site audit. | 1 system | Existing asset | No new cost | Already installed | Existing Darvycom asset | Brisbane Engineering Laboratory |
| Device power supplies | Various | Existing power adapters or internal power supplies used by routers, switches, computers and telephone equipment. | As installed | Existing asset | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |

Hardware subtotal: No new purchasing cost applies because this table records existing resources. Any reuse of the routers and switches is subject to compatibility, condition and capacity testing.

## 5. Existing Software Resources

| Item Type | Vendor | Specifications and Attributes | QTY | Price | Cost | Timeframe | Supplier | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Computer operating systems | Version not recorded | Existing operating systems installed on PCA1–PCA8 and PCB1–PCB8. Versions, patch status and compatibility with the proposed softphone applications must be verified. | 16 installations | Existing licence | No new cost identified | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Computer network drivers | Various | Existing Ethernet adapter drivers that allow the computers to connect to the wired data network. Driver versions require verification. | 16 installations | Included with devices | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Router operating software | Cisco; version not recorded | Provides the current routing and interface configuration for R-A-Current and R-B-Current. Firmware and security support status require verification. | 2 installations | Existing licence | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Switch operating software | Cisco; version not recorded | Provides the current switching configuration for the four existing switches. Support for VLANs, trunks, QoS and security features requires verification. | 4 installations | Existing licence | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Analogue telephone configuration | Vendor not recorded | Existing configuration supporting the legacy copper telephone system and eight analogue telephones. | 1 system | Existing asset | No new cost | Already configured | Existing Darvycom asset | Brisbane Engineering Laboratory |
| TCP/IP networking components | Included with operating systems | Provides basic IPv4 communication between existing computers, switches and routers. | Existing network | Included | No new cost | Already operating | Existing Darvycom asset | Both laboratory blocks |

Software subtotal: No new cost is recorded for existing software. A software and firmware audit must be completed before any existing devices are approved for reuse.

## 6. Existing Other Network Elements

| Item Type | Vendor | Specifications and Attributes | QTY | Price | Cost | Timeframe | Supplier | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Copper telephone cabling | Not recorded | Existing copper cabling used to connect the eight analogue telephone endpoints to the legacy telephone system. | Minimum 8 endpoint connections | Existing asset | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Ethernet cabling | Not recorded | Existing copper Ethernet cabling connecting computers, switches and routers. Cable category and certification results are not documented and must be inspected. | As installed | Existing asset | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Ethernet patch leads | Various | Existing patch cables connecting computers and other network equipment to switches and network outlets. | As installed | Existing asset | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Network outlets | Not recorded | Existing data outlets supporting wired computer connections. Outlet quantity, labelling and condition require confirmation. | As installed | Existing asset | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Patch panels and equipment racks | Not recorded | Existing cable termination and equipment-mounting facilities. Available rack space and port capacity must be confirmed. | As installed | Existing asset | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |
| Internet/WAN connection points | Service provider not recorded | Existing external connection points associated with the two current internet routers. Available bandwidth and service details require verification. | 2 connection points | Existing service | Existing operational cost | Currently active | Current service provider | Laboratory Blocks A and B |
| Electrical outlets | Not recorded | Existing electrical connections used by computers and network devices. Capacity for additional PoE switches, servers and UPS equipment must be checked. | As installed | Existing asset | No new cost | Already installed | Existing Darvycom asset | Both laboratory blocks |

Other network elements subtotal: Existing assets only. Cabling condition, category, labelling, port availability and electrical capacity must be validated during the site inspection.

## 7. Existing Resource Gaps

| Resource or Capability | Existing State | Required Project State | Identified Gap |
| --- | --- | --- | --- |
| Computers | 16 computers | 20 computers | Four additional or replacement computers required |
| Telephones | 8 analogue phones | 20 IP phones | Existing analogue system must be replaced or migrated to VoIP |
| Headsets | No existing quantity documented | 40 headsets | Headsets must be supplied |
| Data servers | No dedicated data servers documented | 2 data servers | Two servers must be provided and configured |
| Voice server | No IP-PBX voice server documented | 1 IP-PBX voice server | A voice server or Cisco CME router must be implemented |
| Video conferencing | No dedicated system documented | 1 video-conferencing system | New video-conferencing endpoint required |
| Switching | 4 existing switches with capabilities not confirmed | 4 managed PoE switches | Existing switches must be assessed and upgraded or replaced |
| VPN | VPN support not documented | Secure VPN-capable routing | VPN-capable routing and configuration required |
| Network segmentation | VLANs not documented | Voice, data, server and management VLANs | VLANs and trunk links must be configured |
| Traffic prioritisation | QoS not documented | Voice and video traffic prioritisation | QoS policies must be implemented |
| Monitoring | No central monitoring documented | PRTG Network Monitor | PRTG monitoring must be deployed and configured |
| Power protection | UPS coverage not documented | UPS protection for core network devices | UPS units or capacity adjustments required |
| Security | Limited existing controls | Firewalling, access control, patching and endpoint protection | Security controls must be strengthened |

## 8. Target Project Specifications and Attributes

| Project Attribute | Required Specification |
| --- | --- |
| Network capacity | Support at least 20 computers, 20 IP phones, 40 headsets, two data servers, one IP-PBX voice server and one video-conferencing system |
| Switching | Four managed switches with sufficient Ethernet ports and PoE support |
| Routing | VPN-capable routing between the laboratory network, remote connections and external services |
| Network segmentation | Separate voice, data, server and management traffic through configured VLANs |
| VLAN design | Use the approved Network 3 VLAN structure, including VLANs 10, 20, 30, 40, 50, 60, 110, 120 and 130 |
| Voice services | IP telephony managed through the IP-PBX/Cisco CME voice server |
| Performance | Low latency, low packet loss and sufficient bandwidth for VoIP, video conferencing, data and DSP activities |
| Quality of Service | Voice and video traffic prioritised over lower-priority network traffic |
| Security | VPN, firewall controls, secure device access, access control, patching, antivirus and endpoint protection |
| Monitoring | PRTG Network Monitor used to monitor availability, bandwidth, utilisation, latency, packet loss and device status |
| Reliability | UPS protection for critical routers, switches and servers, supported by configuration backups and recovery procedures |
| Scalability | Sufficient port, address and bandwidth capacity for future devices and users |
| Testing | Packet Tracer simulation, connectivity tests, VLAN tests, server access tests, VoIP tests, video-conferencing tests and PRTG monitoring validation |
| Deployment | Staged rollout across Laboratory Block A and Laboratory Block B to reduce operational disruption |

## 9. Assumptions and Constraints

- The inventory is based on the approved Darvycom Australia project information and the Network 1 Packet Tracer design.

- The existing network contains two routers, four switches, 16 computers and eight analogue telephones.

- Exact physical device models, serial numbers, software versions, suppliers and original prices have not been provided.

- These missing details must be collected through an onsite network and asset audit.

- Existing switches and routers must not automatically be assumed suitable for the final solution. Their port capacity, PoE support, VLAN functions, QoS capabilities, firmware support and physical condition must be tested.

- Proposed equipment such as IP phones, headsets, servers, the video-conferencing system and PRTG monitoring is not classified as an existing resource.

- All work relates only to the Darvycom Australia Cluster 6 project.