Queue-Based Simulation Control System Report

Project: Darvycom Brisbane Engineering Laboratory Network & DSP Upgrade

Location: Blocks A and B, Brisbane Engineering Laboratory

Project Manager: Leonardo Matias Ciarcia

Document ID: C6-SIM-QUEUE-01

Version: 1.0

Prepared: 18 March 2026

Baseline: 24 weeks / AUD 184,800

# 1. Executive summary

This report designs a discrete-event simulation control system for the output queue of a Darvycom network interface. The model receives voice, video and data packets, assigns them to priority queues, measures occupancy and delay, and changes control actions when thresholds are crossed. The purpose is to test congestion behaviour before deploying QoS settings on Network 3.

Design decision: Use three logical queues: Q1 voice, Q2 video and Q3 data. Apply strict priority to voice with a maximum service budget, weighted service to video/data, threshold alarms and rate control for non-critical traffic.

# 2. System boundary and assumptions

| Element | Model value / role |
| --- | --- |
| Input streams | Voice, video and normal data packet arrivals |
| Server | One monitored output link; test case uses 100 Mbit/s to make congestion visible |
| Packet size | 1500 bytes for the M/M/1 analytical approximation; the full simulation may use traffic-specific distributions |
| Queue capacity | 1000 packets total, with logical class queues |
| Sampling interval | 1 second for controller decisions; event timing remains packet based |
| Outputs | Queue length, utilisation, drop count, delay, jitter estimate and state transitions |

# 3. Queue architecture

| Queue | Traffic | Service rule | Target |
| --- | --- | --- | --- |
| Q1 | VoIP signalling and RTP | Priority service with policing to prevent starvation | Lowest delay and loss |
| Q2 | Video conferencing | Weighted service after Q1 | Stable throughput and bounded delay |
| Q3 | General data and backups | Remaining weighted capacity; rate-controlled in congestion | Throughput without harming real-time traffic |

# 4. Controller states and actions

| State | Queue occupancy | Control action | Exit condition |
| --- | --- | --- | --- |
| GREEN | <50% | Normal weighted scheduling; collect statistics | Occupancy reaches 50% |
| AMBER | 50-75% | Increase real-time priority, warn operator, defer backups | Below 45% or reaches 75% |
| RED | >75% | Rate-limit Q3, enable early drop for data, alarm and preserve Q1/Q2 | Below 65% for five samples |
| RECOVERY | Falling after RED | Gradually restore Q3 bandwidth and verify stability | Below 45% for five samples |

# 5. Control logic

- Generate packet-arrival events for voice, video and data using the selected rates or distributions.

- Classify each packet and append it to Q1, Q2 or Q3. If the capacity is exceeded, record a class-specific drop.

- When the output link becomes available, select the next packet using the configured priority/weighted rule.

- Calculate waiting time as service_start - arrival_time and end-to-end queue time as departure_time - arrival_time.

- Every second, calculate total occupancy, mean/95th-percentile delay, utilisation and drops.

- Apply the GREEN, AMBER, RED or RECOVERY control action and record the state transition.

- Stop after the planned duration and export time-series graphs and a scenario summary table.

## 6. MATLAB-style pseudocode

initialise event_list, Q1, Q2, Q3, state='GREEN'
while simulation_time < duration
    event = remove_next_event(event_list)
    simulation_time = event.time
    if event.type == ARRIVAL: classify_and_enqueue(event.packet)
    if event.type == DEPARTURE: record_delay_and_start_next_packet()
    if event.type == SAMPLE: measure_queue(); apply_state_controller()
end
export queue_length, delay, drops, utilisation, state_history

# 7. Analytical validation

Before running the discrete-event model, validate the link with the M/M/1 approximation. At 100 Mbit/s and 1500-byte packets, service rate mu is approximately 8,333 packets/s. The scenarios below confirm that the controller should react before utilisation reaches 100%.

| Scenario | Arrival rate | rho | Expected queue behaviour | Expected controller state |
| --- | --- | --- | --- | --- |
| S1 normal | 5,000 pkt/s | 0.60 | Short stable queue; low delay | GREEN or brief AMBER |
| S2 busy | 7,500 pkt/s | 0.90 | Queue and delay increase rapidly | AMBER then RED if sustained |
| S3 overload | 9,000 pkt/s | 1.08 | Unstable without drops/rate control | RED |
| S4 recovery | Return to 4,000 pkt/s | 0.48 | Backlog drains and delay falls | RECOVERY then GREEN |

# 8. Simulation acceptance criteria

| Criterion | Pass condition |
| --- | --- |
| Voice protection | Q1 95th-percentile waiting time remains below the project voice queue budget during S2 |
| Controlled overload | S3 produces bounded queue occupancy through rate control or documented drops rather than memory exhaustion |
| Recovery | Controller returns to GREEN after the configured recovery period without rapid state oscillation |
| Fairness | Q2 and Q3 continue receiving service; strict voice priority does not create permanent starvation |
| Observability | Every state change is timestamped and the output includes delay, queue length, drops and utilisation |

# 10. Risks and limitations

The M/M/1 model is an approximation; real packet sizes and arrivals are not perfectly exponential.

Strict priority can starve lower classes unless a service budget or policing rule is enforced.

A home-lab PRTG graph can demonstrate monitoring but cannot replace queue measurements from the simulation or Darvycom production network.

Simulation results must be validated against Packet Tracer configurations and, where possible, measured device performance.

# 11. Conclusion

The proposed queue control system provides a repeatable way to test QoS and congestion decisions for Network 3. It combines discrete-event simulation, analytical queue validation, threshold control and an evidence procedure. The design is ready for implementation and execution in MATLAB/Simulink, SimEvents or an equivalent discrete-event tool.

# 12. Sign-off

| Role | Name | Decision | Date |
| --- | --- | --- | --- |
| Prepared by | Leonardo Matias Ciarcia | Design completed | 18/03/2026 |
| Simulation reviewer | Flavia | Approved / changes required | 18/03/2026 |
| Project sponsor | Dijo | Accepted / rejected | 18/03/2026 |