NETWORK TEST IMPLEMENTATION AND RESULTS REPORT

Project: A-Plus Institute Enterprise Network Upgrade
Network Design: Network Option 2 – Final Selected Network
Prepared by: Leonardo Matias Ciarcia
Role: Network Technical Specialist
Test date: 10/10/2025
Version: 1.0
Status: Test execution record – evidence to be completed

- PURPOSE

This report documents the setup, execution and results of tests performed on the A-Plus Institute final network design.

The tests verify:

- Switch-to-switch connectivity.

- Access-port and VLAN assignments.

- Loop prevention.

- Private VLAN behaviour in a separate supported test scenario.

- Switch virtual interfaces.

- Inter-VLAN routing.

- HSRP redundancy.

- DHCP and DNS.

- Internet and inter-campus connectivity.

- Secure and guest wireless access.

- ACL security.

- IPv4/IPv6 interoperability.

- Network monitoring and configuration persistence.

Actual test results must be entered from the real Packet Tracer, IOS, Wireshark or wireless-tool output. A test must not be marked Pass without corresponding evidence.

- TEST ENVIRONMENT

| Item | Test environment |
| --- | --- |
| Main simulation software | Cisco Packet Tracer |
| Test file | Copy of APlus_Network3_Final.pkt, labelled Network Option 2 in the report |
| Network | A-Plus Institute Brisbane and Rookwood final design |
| Brisbane routing | BRI-RTR-FINAL, BRI-CORE-L3-A and BRI-CORE-L3-B |
| Brisbane access layer | BRI-SW-F1 to BRI-SW-F5 and BRI-SW-SERVER |
| Rookwood routing | ROO-RTR-FINAL and ROO-CORE-L3 |
| Rookwood access layer | ROO-SW-ADMIN and ROO-SW-CLASS |
| Servers | BRI-SRV-01, BRI-NAS-01, BRI-NMS-01, ROO-SRV-01 and ROO-NAS-01 |
| Wireless | Brisbane and Rookwood WLCs, APs, APlus-Secure and APlus-Guest |
| WAN | 3TTECH-ISP, INTERNET-SRV and APLUS-CLOUD-BACKUP |
| Packet analysis | Wireshark on an authorised real network or supported capture source |
| Wireless testing | NetSpot and TamoGraph using actual survey readings |
| IPv6 testing | Separate dual-stack copy of the simulation |

Wireshark, NetSpot and TamoGraph outputs must be identified as separate real-network or field evidence. They must not be described as traffic captured directly from Packet Tracer.

3.TEST EXECUTION AND RESULTS

Test T01 – Device and interface status

Procedure:

- On the routers and Layer 3 core switches, run show ip interface brief.

- Inspect the Packet Tracer topology for red links.

- Confirm that required interfaces are up/up.

Expected result:

All required interfaces are up/up. A redundant Layer 2 uplink may be amber because STP has placed it in a blocking state.

BRI-CORE-L3

BRI-RTR

ROO-RTR

ROO-CORE-L3

Actual result:

Status: ☐ Pass ☐ Fail ☐ Retest required

Test T02 – Switch-to-switch trunk connectivity

Procedure:

- Run show interfaces trunk on both Brisbane core switches and the access switches.

- Confirm that each required uplink operates as a trunk.

- Confirm that Brisbane VLANs 10, 20, 30, 40, 60 and 70 are allowed.

Expected result:

The trunks are operational and carry the required VLANs.

Actual result:

BRI-CORE-L3

Status: ☐ Pass ☐ Fail ☐ Retest required

Test T03 – Access ports and VLAN membership

Procedure:

- Run show vlan brief.

- Inspect the access port used by each representative PC, server and AP.

- Confirm staff ports use VLAN 20 or 120.

- Confirm student ports use VLAN 30 or 130.

- Confirm Brisbane servers and AP management use VLAN 40.

- Confirm Rookwood servers and AP management use VLAN 140.

Expected result:

Each endpoint is assigned to its approved VLAN.

Actual result:

BRI-CORE-L3

Status: ☐ Pass ☐ Fail ☐ Retest required
Evidence: E03 – VLAN and access-port output

Test T04 – Loop-prevention test

Procedure:

- Run show spanning-tree.

- Identify the active and blocked redundant Brisbane uplinks.

- Start a repeated ping from a Brisbane client to its gateway.

- In the test copy, disconnect the currently forwarding uplink.

- Wait for Rapid-PVST to reconverge.

- Repeat the ping and reconnect the original link.

Expected result:

STP prevents a Layer 2 loop. The backup link changes to forwarding and connectivity returns after convergence.

Actual result:

V06 - Loop prevention before failure

V07 – Connectivity after uplink failure

V08 – Restored network

Status: ☐ Pass ☐ Fail ☐ Retest required
Evidence: E04 – STP status and failover screenshots

Test T05 – Private VLAN test

The approved A-Plus design uses standard VLAN separation and ACLs. A private VLAN test must therefore be conducted as a separate isolated assessment scenario so the approved final design is not changed.

Test arrangement:

- One primary private VLAN.

- One isolated secondary VLAN.

- Two isolated host ports.

- One promiscuous port connected to an authorised gateway or server.

Procedure:

- Confirm the primary and isolated VLAN relationship.

- Confirm the host-port and promiscuous-port assignments.

- Ping between the two isolated hosts.

- Ping from each isolated host to the promiscuous gateway or server.

Expected result:

The isolated hosts cannot communicate directly with each other, but each can communicate with the promiscuous port. This is the intended private-VLAN behaviour described by Cisco.

Actual result:

Status: ☐ Pass ☐ Fail ☐ Not executed – tool limitation
Evidence: E05 – Private VLAN configuration and ping results

Important limitation:

Do not claim that a standard guest VLAN ACL is a private VLAN. If the Packet Tracer switch rejects the private-VLAN commands, capture the rejection, record the test as Not executed – tool limitation, and obtain assessor approval to use supported Cisco equipment or another approved lab environment.

Test T06 – SVI and inter-VLAN routing

Procedure:

- Run show ip interface brief on the Layer 3 core switches.

- Confirm the required SVIs are up/up.

- From BRI-ADMIN-PC, ping 10.10.20.1 and 10.10.40.10.

- From BRI-STUDENT-PC4, ping 10.10.30.1 and 10.10.40.10.

- From ROO-STAFF-PC, ping 10.20.20.1 and 10.20.40.10.

Expected result:

Each client reaches its gateway and authorised routed destination.

Status: ☐ Pass ☐ Fail ☐ Retest required