Project: A-Plus Institute – Network Option 2
Prepared by: Leonardo Ciarcia
Test environment: Cisco Packet Tracer
Test date: 10/10/2025

The purpose of this test is to verify VLAN configuration, access and trunk ports, communication within and between authorised VLANs, and loop prevention on the redundant Brisbane switching network.

| Evidence | Screenshot to include | What the evidence must show |
| --- | --- | --- |
| V01 – Network topology | Brisbane topology showing BRI-CORE-L3-A, BRI-CORE-L3-B, floor switches and their uplinks. Include the Rookwood section separately. | Device names and connections clearly identify the network being tested. |
| V02 – VLAN creation | Run show vlan brief on BRI-CORE-L3-A and ROO-CORE-L3. Include the relevant VLAN configuration from show running-config if available. | Brisbane VLANs 10, 20, 30, 40, 60 and 70; Rookwood VLANs 110, 120, 130, 140, 160 and 170. |
| V03 – Access-port configuration | Run show interfaces fastEthernet0/2 switchport on BRI-SW-F1 and BRI-SW-F3. | F1’s client port operates as an access port in VLAN 20. F3’s client port operates as an access port in VLAN 30. |
| V04 – Trunk configuration | Run show interfaces trunk on BRI-SW-F1 and ROO-SW-CLASS. | Uplinks operate as trunks and allow the intended campus VLANs. Both Brisbane uplinks should be represented. |
| V05 – VLAN connectivity | Capture client IP information and the ping tests listed below. | Clients have the correct subnet and gateway, and the tested communication succeeds. |
| V06 – Loop prevention before failure | Run show spanning-tree vlan 20 and show spanning-tree vlan 30 on BRI-SW-F1. | Root information and uplink roles are visible. For each VLAN, the redundant topology has a forwarding path and an alternative blocked path. |
| V07 – Connectivity after uplink failure | Disable the active VLAN 20 uplink on BRI-SW-F1 in the test copy. Capture show spanning-tree vlan 20 and repeat the connectivity test. | The remaining uplink becomes the forwarding path and connectivity recovers after reconvergence. |
| V08 – Restored network | Re-enable the disabled uplink. Capture the STP output and repeat the ping. | Both physical uplinks are restored, STP again controls the redundant paths, and connectivity works. |

V01– Network Topology

V02- VLAN creation Brisbane

V02- VLAN creation Rookwood

V03 – Access-port configuration BRI-SW-F1

V03 – Access-port configuration BRI-SW-F1

V04 – Trunk configuration BRI-SW-F1

V04 – Trunk configuration ROO-SW-CLASS.

V05 – VLAN connectivity:

- BRI-ADMIN-PC, VLAN 20 -> BRI-STAFF-PC’s actual DHCP address, VLAN 20

- BRI-ADMIN-PC, VLAN 20 -> BRI-SRV-01: 10.10.40.10, VLAN 40

- ROO-STAFF-PC, VLAN 120 -> ROO-SRV-01: 10.20.40.10, VLAN 140

V06 - Loop prevention before failure

V07 – Connectivity after uplink failure

V08 – Restored network