# Switch Configuration Troubleshooting Report

## 1. Project information

Project: A-Plus Institute Network Upgrade
Network tested: Network 3 – Final Network Solution
Testing software: Cisco Packet Tracer
Test device: Switch connected to BRI-ADMIN-PC
Test computer: BRI-ADMIN-PC
Test date:10/10/2025
Tested by: Leonardo Ciarcia

## 2. Purpose

The purpose of this test was to check the switch VLAN configuration, port status and Spanning Tree Protocol settings. A controlled switch port problem was created, diagnosed, repaired and retested.

## 3. Initial VLAN check

The following command was entered on the switch:

show vlan brief

The output displayed the VLANs configured on the switch and the access ports assigned to each VLAN. The port connected to BRI-ADMIN-PC was assigned to the correct VLAN.

Figure 1 – VLAN configuration displayed using the show vlan brief command.

Result: Passed. The required VLANs were present and the tested port had the correct VLAN assignment.

## 4. Initial switch port check

The following command was used:

show interfaces status

The output showed that the port connected to BRI-ADMIN-PC was in connected status before the troubleshooting test.

Figure 2 – Initial interface status showing the BRI-ADMIN-PC port connected.

Result: Passed. The switch port was initially active and operational.

## 5. STP settings check

The following command was entered:

show spanning-tree

The output showed the STP root, designated and forwarding port information. The STP configuration was operating correctly and preventing switching loops.

Figure 3 – Spanning Tree Protocol status and switch port roles.

Result: Passed. STP was active and no switching loop was detected.

## 6. Controlled switch fault

To simulate a switch configuration problem, the port connected to BRI-ADMIN-PC was disabled using the following commands:

configure terminal

interface fa0/X

shutdown

end

Note: fa0/X was replaced with the actual port connected to BRI-ADMIN-PC.

The interface status was checked again:

show interfaces status

The port displayed as disabled or down. A ping was then sent from BRI-ADMIN-PC:

ping 10.10.20.1

The ping failed because the computer’s switch port was disabled.

Figure 4 – Switch port showing disabled or down status.

Figure 5 – Failed ping while the switch port was disabled.

Result: Failed as expected. The controlled fault successfully interrupted connectivity.

## 7. Corrective action

The problem was diagnosed using the show interfaces status command. The interface was administratively disabled.

The following commands were used to repair it:

configure terminal

interface fa0/X

no shutdown

end

After waiting for the link to become green, the interface was checked again:

show interfaces status

The port returned to connected status.

Connectivity was retested from BRI-ADMIN-PC:

ping 10.10.20.1

ping 10.10.40.10

Both destinations responded successfully.

Figure 6 – Successful connectivity tests after the repair.

Result: Passed. The port and network connectivity were successfully restored.

## 8. Troubleshooting log

| Item checked | Finding | Action taken | Retest result | Status |
| --- | --- | --- | --- | --- |
| VLAN configuration | Required VLANs and port assignments were displayed | No change required | VLAN configuration remained operational | Passed |
| Initial port status | BRI-ADMIN-PC port was connected | No change required | Initial connectivity was available | Passed |
| STP settings | STP was active and showed the required port roles | No change required | No switching loop was detected | Passed |
| Controlled port fault | BRI-ADMIN-PC port was administratively disabled | Investigated with show interfaces status | Failed ping confirmed the fault | Identified |
| Port correction | Interface was disabled with the shutdown command | Applied the no shutdowncommand | Port returned to connected status | Resolved |
| Final connectivity | Network communication was restored | Pings were repeated | 10.10.20.1 and 10.10.40.10responded | Passed |

## 9. Final outcome

The switch troubleshooting test was completed successfully. The VLAN configuration, interface status and STP settings were checked. A controlled disabled-port problem was identified using Cisco IOS diagnostic commands and corrected with the no shutdown command. Final ping tests confirmed that connectivity had been restored.

## 10. Evidence summary

The following evidence was collected:

- VLAN configuration using show vlan brief.

- Initial connected port using show interfaces status.

- STP configuration using show spanning-tree.

- Disabled switch port and failed ping.

- Restored switch port and successful final pings.