Project: A-Plus Institute – Cluster 5
Unit: ICTNWK612
Solution: Network Option 2 (Network 3 in Packet Tracer)
Prepared by: Leonardo Ciarcia, Assistant IT Manager, HV Technologies
Monitoring date: 11/10/2025

- Purpose

Monitor IP connectivity and performance across the Brisbane and Rookwood campuses. Checks cover local gateways, campus servers, the inter-campus connection and the simulated Internet service.

The following targets continue the earlier monitoring plan. They are proposed project targets, not confirmed measurements or an approved service-level agreement.

- Performance metrics

| Metric | Proposed target | Measurement method |
| --- | --- | --- |
| Local network response time | Average round-trip time ≤10 ms | Ping local gateway and campus server |
| Inter-campus response time | Average round-trip time ≤100 ms | Ping the remote campus server |
| External response time | Average round-trip time ≤150 ms | Ping INTERNET-SRV: 209.165.200.10 |
| Packet loss | Operational target ≤1%; expect 0% in the short lab checks | Record packets sent, received and lost |
| Service availability | ≥99.9% per month | Time-based service monitoring and outage records |
| Link utilisation | Normally <70%; warning 70% to below 85%; critical ≥85% when sustained | Interface traffic measurements |
| Interface condition | Required connected links up; no increase in error counters during the observation period | Switch/router interface output |

Ping provides reachability, packet-loss and round-trip-time observations.

- Monitoring method and schedule

Use Packet Tracer for the assessment checks. Record a baseline, repeat the same checks approximately five minutes later, and retest after any corrective action. Keep the same source devices and destinations.

The proposed operational schedule remains: reachability checks every minute, interface measurements every five minutes, continuous collection of configured syslog messages, twice-daily review, weekly trend review and monthly availability/capacity review. These are deployment activities; automatic polling has not been demonstrated in this Packet Tracer exercise.

Packet Tracer is a network simulator. Label its results as simulation evidence; short ping checks do not establish monthly availability or real network capacity.

- Performance monitoring sheet

Baseline date/time: [Enter]
Follow-up date/time: [Enter]

For each result, enter: packets received/sent; loss percentage; average RTT in milliseconds. Use N/A for RTT if no reply is received.

| ID | Source | Destination | Baseline result |
| --- | --- | --- | --- |
| M01 | BRI-ADMIN-PC | Brisbane gateway: 10.10.20.1 | Successfull |
| M02 | BRI-ADMIN-PC | BRI-SRV-01: 10.10.40.10 | Successfull |
| M03 | BRI-ADMIN-PC | ROO-SRV-01: 10.20.40.10 | Successfull |
| M04 | BRI-ADMIN-PC | INTERNET-SRV: 209.165.200.10 | Successfull |
| M05 | ROO-STAFF-PC | ROO-SRV-01: 10.20.40.10 | Successfull |

For M03, a successful ping demonstrates inter-campus reachability. Check VPN status separately if claiming that the traffic was encrypted.