NETWORK CONFIGURATION & TOPOLOGY

Prepared by: Leonardo Matias Ciarcia
Student ID: CTI20221062
Prepared for: A-Plus Institute
Date: 07/06/2026
Version: 1.0

1. Current topology map

2. Network Diagram (Current – Packet Tracer)

ZOOM1:

ZOOM2:

ZOOM3:

### Brisbane device placement

| Area | Devices |
| --- | --- |
| Existing Level 1 | BRI-RTR, BRI-SW, BRI-SRV, BRI-ADMIN-PC1 |
| Existing Level 2 | BRI-STAFF-PC1 |
| Existing Level 3 | BRI-AP, BRI-STUDENT-PC1, BRI-LAPTOP1 |

The three levels share one central switch and one autonomous access point. This represents the current coverage and capacity limitations.

### Rookwood device placement

- ROO-RTR

- ROO-SW

- ROO-AP

- ROO-STAFF-PC1

- ROO-LAPTOP1

Rookwood is a separate campus, not another Brisbane level.

## 3. Link details

### Carrier and external links

| From | Port | To | Port | Link purpose |
| --- | --- | --- | --- | --- |
| 3TTECH-ISP | G0/0 | BRI-RTR | G0/0 | Brisbane carrier connection |
| 3TTECH-ISP | G0/1 | ROO-RTR | G0/0 | Existing Rookwood carrier connection |
| 3TTECH-ISP | G0/2 | ISP-SW | G0/1 | External services network |
| ISP-SW | Fa0/1 | INTERNET-SRV | Fa0 | Simulated Internet service |

Use Copper Straight-Through or Automatic. Router ports require no shutdown.

### Brisbane internal links

| From | Port | To | Port | Link type |
| --- | --- | --- | --- | --- |
| BRI-RT | G0/1 | BRI-SW | G0/1 | LAN uplink |
| BRI-SW | Fa0/2 | BRI-SRV | Fa0 | Server connection |
| BRI-SW | Fa0/3 | BRI-AP | Port 0 | Autonomous AP connection |
| BRI-SW | Fa0/4 | BRI-ADMIN-PC1 | Fa0 | Wired client |
| BRI-SW | Fa0/5 | BRI-STUDENT-PC1 | Fa0 | Wired client |
| BRI-SW | Fa0/6 | BRI-STAFF-PC1 | Fa0 | Wired client |
| BRI-AP | Wireless | BRI-LAPTOP1 | Wireless0 | Wireless connection |

### Rookwood internal links

| From | Port | To | Port | Link type |
| --- | --- | --- | --- | --- |
| ROO-RTR | G0/1 | ROO-SW | G0/1 | LAN uplink |
| ROO-SW | Fa0/2 | ROO-AP | Port 0 | Autonomous AP |
| ROO-SW | Fa0/3 | ROO-STAFF-PC1 | Fa0 | Wired client |
| ROO-SW | Fa0/4 | ROO-STUDENT-PC1 | Fa0 | Wired client, if included |
| ROO-AP | Wireless | ROO-LAPTOP1 | Wireless0 | Wireless connection |

The laptops do not require Ethernet cables.

## 4. Network addressing

| Network segment | Network/addressing |
| --- | --- |
| Brisbane WAN | 203.0.113.0/30 |
| 3TTECH-ISP G0/0 | 203.0.113.1 |
| BRI-RTR G0/0 | 203.0.113.2 |
| Brisbane LAN | 192.168.10.0/24 |
| Brisbane gateway | 192.168.10.1 |
| Brisbane server | 192.168.10.10 |
| Brisbane clients | DHCP from 192.168.10.21 |
| Rookwood WAN | 198.51.100.0/30 |
| 3TTECH-ISP G0/1 | 198.51.100.1 |
| ROO-RTR G0/0 | 198.51.100.2 |
| Rookwood LAN | 192.168.20.0/24 |
| Rookwood gateway | 192.168.20.1 |
| Rookwood clients | DHCP from 192.168.20.21 |
| External-service network | 209.165.200.0/24 |
| External gateway | 209.165.200.1 |
| Internet server | 209.165.200.10 |

The AccessPoint-PT devices do not show a configurable management IP in your Packet Tracer version. They operate as Layer 2 wireless bridges.

## 5. Carrier information

| Item | Current-network information |
| --- | --- |
| Carrier/service provider | 3TTech |
| Service | Business Internet/WAN connectivity |
| Brisbane carrier subnet | 203.0.113.0/30 |
| Rookwood carrier subnet | 198.51.100.0/30 |
| External-services subnet | 209.165.200.0/24 |
| Connection type | Simulated Ethernet WAN connection |
| Brisbane status | Existing operational Internet connection |
| Rookwood status | Existing but limited/unreliable carrier connection |
| Inter-campus connectivity | No site-to-site VPN |
| Security | Basic NAT at each site router |
| Redundancy | No redundant WAN or core connection |
| Carrier limitation | Failure of one carrier link disconnects that campus |

## 6. Current-network limitations

- One central switch supports all three existing Brisbane levels.

- Only one autonomous AP provides Brisbane wireless access.

- The network is flat and has no VLAN separation.

- Brisbane and Rookwood cannot communicate through private addresses.

- There is no site-to-site VPN.

- There is no WLAN controller.

- There is no redundant router, core switch or carrier link.

- Wireless capacity and coverage are insufficient for the two new floors.

- Rookwood’s existing carrier service is limited or unreliable.

- The network has limited monitoring, centralised security and scalability.