PLAN AND MANAGE TROUBLESHOOTING ADVANCED INTEGRATED IP NETWORKS

TROUBLESHOOTING STRATEGY PLAN

Project: A-Plus Institute Enterprise Network Upgrade
Network Design: Network Option 2 – Final Selected Network
Prepared by: Leonardo Matias Ciarcia
Role: Network Technical Specialist
Date: 2 October 2025
Version: 1.0
Status: Submitted for review

- PURPOSE

The purpose of this document is to establish a structured troubleshooting strategy for the A-Plus Institute advanced IP network. The strategy is designed to identify, isolate, diagnose and resolve network faults while minimising disruption to staff, students and business services.

The strategy supports the institute’s growth, including 15 additional employees, two new classroom levels at the Brisbane campus, the development of the Rookwood campus and secure communication between both locations.

- NETWORK SCOPE

The troubleshooting strategy covers:

- The five-level Brisbane campus, consisting of three existing levels and two new classroom levels.

- The server room located on Existing Level 1.

- The Rookwood campus network.

- The 3TTech ISP connection.

- BRI-RTR-FINAL and ROO-RTR-FINAL routers.

- BRI-CORE-L3-A and BRI-CORE-L3-B redundant Layer 3 core switches.

- Brisbane floor switches BRI-SW-F1 to BRI-SW-F5.

- The Brisbane server switch, servers, NAS, monitoring server and WLC.

- Rookwood core and access switches, servers, NAS, WLC and wireless AP.

- Wired staff, student and management devices.

- APlus-Secure corporate wireless and APlus-Guest wireless.

- VLANs, HSRP, Rapid-PVST, DHCP, DNS, NAT, ACLs and the site-to-site IPsec VPN.

The main network segments are:

| Location | VLANs and functions |
| --- | --- |
| Brisbane | VLAN 10 Management, VLAN 20 Staff, VLAN 30 Students, VLAN 40 Servers, VLAN 60 Corporate Wi-Fi and VLAN 70 Guest Wi-Fi |
| Rookwood | VLAN 110 Management, VLAN 120 Staff, VLAN 130 Students, VLAN 140 Servers, VLAN 160 Corporate Wi-Fi and VLAN 170 Guest Wi-Fi |

- PERFORMANCE GOALS

The following figures are proposed service targets. They must be confirmed against real baseline measurements after implementation and must not be presented as results already achieved.

| Performance area | Proposed goal |
| --- | --- |
| Critical network availability | At least 99.9% availability during scheduled campus operating hours |
| Interface condition | All required interfaces remain up/up, with no unexplained link flapping |
| Local LAN latency | Average round-trip time of 10 ms or less |
| Brisbane–Rookwood latency | Average round-trip time of 100 ms or less through the VPN |
| Internet latency | Average round-trip time of 150 ms or less to an approved external target |
| Packet loss | 1% or less during normal operation |
| Jitter | 30 ms or less for delay-sensitive traffic |
| Link utilisation | Normally below 70%; warning at 70–84%; critical at 85% or more when sustained |
| Device CPU use | Normally below 70%; critical above 85% when sustained |
| Device memory use | Normally below 75%; critical above 90% when sustained |
| DHCP service | A valid address, gateway and DNS information supplied within 10 seconds |
| DNS and internal service response | Successful internal response within approximately 2 seconds under normal conditions |
| Redundancy recovery | Connectivity restored within 60 seconds after a single approved uplink or core-path failure |
| Wireless coverage | Target of at least –67 dBm RSSI and at least 25 dB SNR in required teaching and administration areas |
| Security enforcement | Guest clients can access the internet but cannot access staff, student, management or server networks |
| Monitoring coverage | All critical routers, switches, servers, WLCs and APs are visible to the monitoring process |

- POTENTIAL NETWORK ISSUES

| Potential issue | Likely symptoms | Initial checks | Planned corrective action |
| --- | --- | --- | --- |
| Damaged cable, incorrect cable or powered-off interface | Red link, no connectivity or interface down/down | Check power, cable, port and show ip interface brief | Reconnect or replace the cable, use the correct port and apply no shutdown where required |
| Incorrect IP address, subnet mask or gateway | Local or remote ping fails | Check ipconfig /all, addressing plan and gateway | Correct the client or interface configuration and renew DHCP |
| Incorrect access VLAN | Device receives an address from the wrong subnet or cannot reach its gateway | Use show vlan brief and inspect the access port | Place the port in the correct staff, student, server or management VLAN |
| Missing VLAN on a trunk | One or more VLANs fail across a switch uplink | Use show interfaces trunk | Add the required VLAN to the trunk allowed list |
| Spanning Tree problem or Layer 2 loop | Broadcast storm, instability or multiple interfaces changing state | Use show spanning-tree | Correct the topology and Rapid-PVST configuration. An intentionally blocked redundant uplink is normal |
| HSRP failure | Virtual gateway unavailable or incorrect active core | Use show standby brief on both Brisbane core switches | Restore the failed SVI or core link and verify HSRP priorities and pre-emption |
| DHCP failure | Client shows 0.0.0.0 or an automatic private address | Check DHCP pools, VLAN, SVI and ip helper-address | Correct the DHCP pool, relay address, excluded addresses or server service |
| Routing failure | Local VLAN works but remote networks fail | Use ping, traceroute and show ip route | Correct the missing or incorrect route and verify routed links |
| VPN failure | Each campus has local access but cannot reach the other campus | Check WAN reachability and IPsec security associations | Correct peer addressing, pre-shared-key mismatch, transform set or mirrored VPN ACL |
| NAT or internet failure | Internal services work but external access fails | Check default routes, NAT interfaces and NAT ACL | Correct inside/outside assignment, NAT rule or ISP route |
| DNS or server service failure | IP ping works but hostname or application access fails | Test the server by IP, use nslookup and check service status | Restore DNS, DHCP, HTTP, file or NAS service and verify server addressing |
| AP does not join WLC | SSID unavailable on one level | Check AP management VLAN, DHCP address and WLC connectivity | Correct the AP port, native VLAN, management address or WLC configuration |
| Wireless coverage or interference issue | Weak signal, unstable connection or low throughput | Use NetSpot or TamoGraph to inspect RSSI, SNR and channels | Adjust AP placement, channel, transmit power or investigate physical obstructions |
| ACL configuration issue | Authorised traffic is blocked or guest traffic reaches internal resources | Inspect ACL entries, direction and counters | Correct the ACL order, source/destination range or interface direction |
| Congestion or interface errors | Slow applications, loss, retransmissions or high delay | Review utilisation, error counters and Wireshark capture | Identify high-use traffic, faulty interfaces or capacity limitations and apply an approved correction |
| Monitoring failure | Device is operating but no statistics or logs are received | Test ICMP, SNMP and syslog reachability | Restore the monitoring path or correct the device’s monitoring configuration |
| Unauthorised configuration change | Unexpected behaviour after maintenance | Compare the running configuration with the approved version | Restore the approved configuration and record the change |

- TROUBLESHOOTING METHOD

The following structured process will be followed for every incident.

Step 1 – Obtain authorisation and protect the environment

Confirm the approved scope, affected services and maintenance window. Save the current configuration and ensure a rollback copy is available before changing a router, switch, WLC or server.

Step 2 – Gather information

Record:

- Who is affected.

- Which campus, level, VLAN, device or application is affected.

- When the problem started.

- Whether the fault is continuous or intermittent.

- Whether any recent configuration, software, cabling or equipment change occurred.

- The exact error message or user symptom.

- Whether the problem can be reproduced.

Step 3 – Define the problem

Compare actual behaviour with the performance goals and the established baseline. Distinguish a genuine fault from expected behaviour. For example, one redundant switch uplink may appear amber because Spanning Tree has placed it in a blocking state to prevent a loop.

Step 4 – Determine the impact and priority

Classify the incident as critical, high, medium or low according to the number of users affected, the importance of the service and the availability of a backup path.

Step 5 – Isolate the fault

Narrow the fault progressively:

Campus → building level → VLAN → switch → interface → endpoint or application.

The VLAN structure allows staff, student, server, management and guest networks to be tested separately without unnecessarily interrupting the entire network.

Step 6 – Select the troubleshooting approach

| Situation | Preferred approach |
| --- | --- |
| Red link or device not powered | Bottom-up, beginning at the physical layer |
| Application or website failure | Top-down, beginning with the service and working toward connectivity |
| IP address or gateway problem | Divide-and-conquer, beginning at Layer 3 |
| Brisbane–Rookwood or internet problem | Follow the path from client to gateway, router, WAN and destination |
| One device behaves differently from similar devices | Compare configurations and spot the differences |
| Suspected cable, port or client problem | Move the problem by using a known-good cable, port or authorised test device |

Step 7 – Test one hypothesis at a time

Develop the most likely cause based on the evidence. Where possible, reproduce the problem in Cisco Packet Tracer before changing the operational environment. Change only one variable at a time and document the result.

Step 8 – Implement or reverse the change

Apply the approved correction. If the change does not solve the problem or causes an additional fault, restore the saved configuration and continue testing the next hypothesis.

Step 9 – Verify full operation

Confirm that the original problem has been resolved and that the correction has not affected another VLAN, service, wireless network, campus or security control.

Step 10 – Document and close

Record the cause, evidence, change, test results, outage duration and preventive recommendation. Escalate unresolved WAN faults to 3TTech and hardware or software faults to the appropriate supplier.

- NETWORK SEGMENT ISOLATION PLAN

| Fault scope | Isolation procedure |
| --- | --- |
| One endpoint | Test its IP settings, cable, wireless association and switch port; compare with a working endpoint in the same VLAN |
| One Brisbane level | Test the local access switch, AP and both redundant core uplinks; do not disconnect other levels |
| One VLAN | Test the access ports, trunk allowance, SVI, HSRP gateway, DHCP relay and applicable ACL |
| One wireless SSID | Test APlus-Secure and APlus-Guest separately and verify the mapped VLAN |
| One server | Test the server address and service from an authorised client in the server VLAN before testing across routed networks |
| One campus | Confirm local services first, followed by the campus router, ISP path and VPN |
| Inter-campus communication | Verify local gateways at both sites, WAN interfaces, VPN ACLs and IPsec counters |
| Internet access | Verify the client gateway, campus default route, NAT, ISP link and approved external target |

Only the smallest necessary segment will be disconnected or changed. High-impact testing must occur during an approved maintenance window, and stakeholders must be informed before service interruption.

- TOOLS AND DIAGNOSTIC COMMANDS

The following tools will support the strategy:

- Cisco Packet Tracer for configuration simulation, packet-path inspection and controlled failure testing.

- Wireshark for authorised live packet capture and protocol analysis.

- NetSpot for quick wireless discovery, signal and channel checks.

- TamoGraph for detailed wireless surveys, heatmaps and RF planning.

- BRI-NMS-01 for ICMP, read-only SNMP and syslog monitoring.

- Cisco IOS diagnostic commands for direct device inspection.

Common commands include:

| Command or test | Purpose |
| --- | --- |
| show ip interface brief | Verify interface addressing and operational state |
| show vlan brief | Confirm VLAN existence and access-port membership |
| show interfaces trunk | Confirm trunk state and allowed VLANs |
| show spanning-tree | Identify root paths, forwarding ports and expected blocked links |
| show standby brief | Verify HSRP active and standby roles |
| show ip route | Inspect routes and default paths |
| show access-lists | Inspect security rules and match counters |
| show crypto isakmp sa | Check IPsec phase-one status |
| show crypto ipsec sa | Check IPsec packet encapsulation and decapsulation |
| ping | Test IP reachability, delay and packet loss |
| traceroute or tracert | Identify the path and possible point of failure |
| ipconfig /all | Verify client address, gateway, DNS and DHCP information |
| nslookup | Test DNS resolution |

- FINAL VERIFICATION SEQUENCE

After a correction, the following regression tests will be completed:

- Confirm required interfaces are up/up.

- Verify HSRP active and standby roles on both Brisbane core switches.

- Verify VLANs, trunks and Rapid-PVST status.

- Confirm staff and student clients receive addresses from the correct DHCP pools.

- From BRI-ADMIN-PC, test 10.10.20.1, 10.10.40.10 and 209.165.200.10.

- From a Brisbane student PC, test the student gateway and BRI-SRV-01.

- From ROO-STAFF-PC, test 10.20.20.1 and 10.20.40.10.

- Test Brisbane-to-Rookwood communication through the site-to-site VPN.

- Confirm that IPsec encapsulation and decapsulation counters increase.

- Confirm that APlus-Secure receives a corporate wireless address and reaches authorised resources.

- Confirm that APlus-Guest reaches the internet but cannot reach internal networks.

- Test access to the approved internet service at 209.165.200.10.

- Perform an approved redundancy test and confirm that the backup uplink becomes forwarding.

- Save the corrected configurations and supporting evidence.

- INCIDENT PRIORITY AND ESCALATION

| Priority | Example | Response target | Escalation |
| --- | --- | --- | --- |
| Critical | Complete campus outage, both core paths unavailable, critical server failure or VPN failure affecting operations | Immediate investigation; initial response within 15 minutes | Project manager, senior network engineer and 3TTech where applicable |
| High | One level, VLAN or major service unavailable | Initial response within 30 minutes | Network supervisor if not restored within one hour |
| Medium | One AP, isolated group or degraded performance | Same business day | Escalate if repeated or if performance continues to decline |
| Low | Single-user issue with an available workaround | According to service-desk schedule | Escalate if unresolved or affecting additional users |