IP NETWORK MONITORING PLAN

Project: A-Plus Institute Enterprise Network Upgrade
Network Design: Network Option 2 – Final Selected Network
Prepared by: Leonardo Matias Ciarcia
Role: Network Technical Specialist
Date: 3 October 2025
Version: 1.0
Status: Submitted for review

- PURPOSE

This monitoring plan defines how the A-Plus Institute IP network will be monitored to maintain performance, availability, security and reliability. It provides monitoring objectives, devices and services to be observed, performance thresholds, tools, schedules, alert handling and reporting requirements.

The plan covers the Brisbane five-level campus and Rookwood campus, including the secure site-to-site VPN provided through 3TTech.

- MONITORING GOALS

The monitoring process will:

- Detect device, interface, service and VPN failures as early as possible.

- Maintain reliable access for staff, students and authorised wireless users.

- Identify congestion, packet loss, excessive latency, interface errors and wireless interference.

- Confirm that both Brisbane core switches and redundant uplinks operate correctly.

- Verify DHCP, DNS, server, NAS and internet availability.

- Confirm that APlus-Secure and APlus-Guest remain mapped to the correct VLANs.

- Detect unexpected configuration changes or security events.

- Maintain historical information for troubleshooting and capacity planning.

- Support the increased number of employees, students and devices.

- provide evidence for maintenance, incident investigation and management review.

- MONITORING STRUCTURE

BRI-NMS-01, located in the Brisbane server VLAN at 10.10.40.40, will represent the central monitoring server. It will collect information using ICMP, read-only SNMP and syslog.

Rookwood devices will be monitored through the secure site-to-site VPN. If all Rookwood devices become unavailable at the same time, the WAN and VPN paths will be investigated before individual Rookwood devices are treated as failed.

The monitoring process will use both:

- Passive monitoring, including SNMP information, interface counters, syslog events and authorised packet captures.

- Active monitoring, including ping, service checks, DNS requests and approved wireless tests.

- DEVICES AND SERVICES TO BE MONITORED

| Component | Examples | Information monitored |
| --- | --- | --- |
| Campus routers | BRI-RTR-FINAL and ROO-RTR-FINAL | WAN and LAN interface state, reachability, CPU, memory, routing, NAT, VPN state and syslog |
| Brisbane core switches | BRI-CORE-L3-A and BRI-CORE-L3-B | SVIs, HSRP status, routing, trunks, STP roles, interface utilisation, errors and configuration changes |
| Brisbane access switches | BRI-SW-F1 to BRI-SW-F5 and BRI-SW-SERVER | Uplink state, access-port state, allowed VLANs, STP condition, utilisation, errors and management reachability |
| Rookwood switches | ROO-CORE-L3, ROO-SW-ADMIN and ROO-SW-CLASS | Interface state, VLANs, trunks, utilisation, errors, routes and management reachability |
| Brisbane servers | BRI-SRV-01, BRI-NAS-01 and BRI-NMS-01 | Reachability, DHCP, DNS, file services, storage availability and system events |
| Rookwood servers | ROO-SRV-01 and ROO-NAS-01 | Reachability, local services, storage and access through the VPN |
| Wireless controllers | BRI-WLC-01 and ROO-WLC-01 | Reachability, AP registration, SSID availability, clients and management interface |
| Wireless APs | BRI-AP-F1 to BRI-AP-F5 and ROO-AP-01 | Reachability, association, channel use, signal, interference and client count |
| Virtual gateways | 10.10.x.1 and 10.20.x.1 gateway addresses | Availability and response time |
| Internet test service | 209.165.200.10 in the simulation | External reachability, DNS and HTTP availability |
| Cloud backup representation | 209.165.200.20 in the simulation | Reachability and approved backup-service availability |
| Security controls | Guest ACLs, corporate WLAN and VPN | Allowed traffic, blocked traffic, unexpected access and VPN status |

- MONITORING TOOLS

| Tool or method | Monitoring use |
| --- | --- |
| BRI-NMS-01 | Central collection point for network status, alerts and historical information |
| ICMP ping | Tests availability, round-trip time and packet loss |
| Read-only SNMP | Collects device, interface, utilisation, CPU, memory and error information |
| Syslog | Receives device events, interface changes, security events and configuration messages |
| Cisco IOS commands | Confirms current routing, HSRP, STP, VLAN, trunk, ACL and VPN conditions |
| Cisco Packet Tracer | Validates the monitoring logic and tests controlled failure scenarios in simulation |
| Wireshark | Investigates packet loss, retransmissions, DHCP, DNS and protocol-specific problems |
| NetSpot | Performs quick wireless signal, SNR, channel and coverage checks |
| TamoGraph | Produces detailed wireless surveys, predictive models and coverage reports |

- PERFORMANCE THRESHOLDS

These are initial proposed thresholds. They will be adjusted after collecting an approved operational baseline.

| Metric | Normal | Warning | Critical |
| --- | --- | --- | --- |
| Device reachability | Successful checks | One missed check | Three consecutive missed checks or confirmed outage |
| Required interface | Up/up and stable | Unexpected state change | Required primary and backup paths unavailable |
| Link utilisation | Below 70% | 70–84% for 15 minutes | 85% or more for 5 minutes |
| Packet loss | 0–1% | Above 1% to 5% | Above 5% |
| Local LAN latency | 10 ms or less | Above 10 ms or significant baseline increase | Repeated timeout or more than twice the normal baseline |
| Inter-campus latency | 100 ms or less | Above 100 ms | Repeated timeout or VPN unavailable |
| Jitter | 30 ms or less | 31–50 ms | Above 50 ms |
| CPU utilisation | Below 70% | 70–85% for 15 minutes | Above 85% for 5 minutes |
| Memory utilisation | Below 75% | 75–90% | Above 90% |
| Interface errors/discards | Below 1% | 1–3% | Above 3% or rapidly increasing |
| Server or NAS storage | Below 75% | 75–90% | Above 90% |
| Wireless RSSI | At least –67 dBm | Below –67 to –75 dBm | Below –75 dBm or service unavailable |
| Wireless SNR | At least 25 dB | 20–24 dB | Below 20 dB |
| HSRP | One active and one standby device | Unexpected role change | Virtual gateway unavailable |
| VPN | Established when required | Repeated re-establishment | Tunnel unavailable or no encrypted traffic passing |
| Guest isolation | Internal access denied | Unexpected ACL match or policy variation | Guest access to protected internal resources |

- MONITORING SCHEDULE

| Frequency | Monitoring activity | Responsible person | Record produced |
| --- | --- | --- | --- |
| Continuous | Receive syslog events from critical routers and switches | Monitoring system/network administrator | Event and alert log |
| Every 1 minute | Ping site routers, Brisbane cores, critical servers, virtual gateways and VPN-dependent targets | BRI-NMS-01 | Availability and latency record |
| Every 5 minutes | Poll interfaces, utilisation, CPU, memory and error counters using read-only SNMP | BRI-NMS-01 | Performance data |
| Every 15 minutes | Test DHCP, DNS, HTTP and approved server availability | Monitoring system | Service status record |
| Hourly during operating hours | Review WLC, AP, SSID and wireless client status | Network administrator | Wireless status entry |
| Twice daily | Review critical alerts at the beginning and near the end of the operating day | Network administrator | Daily monitoring checklist |
| Weekly | Review utilisation, loss, latency, interface errors and recurring events | Network administrator | Weekly technical summary |
| Weekly | Save and compare authorised device configurations | Network administrator | Configuration backup and change record |
| Monthly | Review capacity, uptime, incidents, WAN performance and recurring faults | Network administrator and project manager | Monthly performance report |
| Quarterly | Conduct an approved redundancy and restoration test | Network team | Redundancy test report |
| Quarterly | Complete a detailed wireless check using NetSpot or TamoGraph | Wireless/network specialist | Wireless survey report |
| After any major change | Repeat connectivity, security, VPN, DHCP, wireless and redundancy tests | Change implementer | Post-change validation record |
| After an incident | Review the relevant historical data and confirm recovery | Incident owner | Incident closure report |

- BASELINE PROCESS

During the first 10 business days following implementation, the network team will collect data during both busy and low-use periods. The baseline will include:

- Interface utilisation.

- Local and inter-campus latency.

- Packet loss and jitter.

- CPU and memory utilisation.

- Server response times.

- Wireless RSSI, SNR and channel use.

- Typical numbers of wired and wireless clients.

- VPN availability.

- Normal HSRP and Spanning Tree states.

The baseline will describe normal network behaviour. Alert thresholds may then be adjusted with stakeholder approval to reduce false alarms without hiding genuine performance problems.

- ALERT AND ESCALATION PROCESS

| Alert level | Condition | Required action |
| --- | --- | --- |
| Critical | Campus router, both Brisbane core paths, critical server, virtual gateway or VPN unavailable | Notify the network administrator immediately, begin troubleshooting and escalate to the project manager or 3TTech as applicable |
| High | One level, VLAN, core path or important service unavailable | Investigate within 30 minutes and provide an update if not restored within one hour |
| Medium | Sustained utilisation, packet loss, latency, interface errors or weak wireless performance | Investigate during the same business day and schedule corrective work |
| Informational | Planned failover, successful recovery, routine login or minor state change | Record and review during the daily check |

Every alert must include the affected device, interface or service; time detected; severity; measured value; related events; assigned person; action taken; and closure time.

- REPORTING

The following reports will be prepared:

- Daily alert and incident log for the technical team.

- Weekly technical report showing device availability, utilisation, errors, latency, packet loss and unresolved issues.

- Monthly management summary showing uptime, major incidents, capacity trends, security events and recommendations.

- Post-incident report for every critical outage.

- Post-change report confirming that the network still satisfies connectivity, redundancy and security requirements.

- Quarterly wireless survey and redundancy test report.

Raw monitoring records should be retained for at least 90 days and summary reports for 12 months, subject to the institute’s approved data-retention policy.

- SECURITY AND DATA PROTECTION

Monitoring access will be restricted to authorised personnel. Read-only monitoring access will be used wherever possible. Passwords, wireless keys and SNMP credentials will not be included in reports or screenshots submitted as general evidence.

Monitoring reports and packet captures may contain IP addresses and user traffic information. They must be stored securely, shared only with authorised people and removed according to the approved retention policy. Packet captures will be limited to the necessary interface, time and protocol.

- LIMITATIONS

Packet Tracer demonstrates logical connectivity, security behaviour and controlled failure scenarios, but it does not prove actual production capacity or physical wireless coverage. Real performance conclusions must be supported by authorised measurements from the implemented network.

NetSpot and TamoGraph results must use the correct floor plan, actual measurement locations and genuine readings. Proposed or illustrative results must be clearly labelled and must not be reported as measurements.