NETWORK TESTING TOOLS AND SOFTWARE SELECTION SHEET

Project: A-Plus Institute Enterprise Network Upgrade
Network Design: Network Option 2 – Final Selected Network
Prepared by: Leonardo Matias Ciarcia
Role: Network Technical Specialist
Date: 6 September 2026
Version: 1.0
Status: Submitted for approval

- PURPOSE

The purpose of this selection sheet is to research, compare and select suitable software for testing and troubleshooting the A-Plus Institute network.

The selected software must support:

- Logical network design and pre-deployment simulation.

- Testing of VLANs, SVIs, HSRP, Rapid-PVST, routing, NAT and ACLs.

- DHCP, DNS, internet and site-to-site VPN testing.

- Packet and protocol analysis.

- Wireless coverage and interference assessment.

- Testing of the five-level Brisbane campus and Rookwood campus.

- Production of screenshots, captures, heatmaps and reports as assessment evidence.

- Safe testing without unnecessary interruption to users.

- TOOLS RESEARCHED

Cisco Packet Tracer

Cisco Packet Tracer is a virtual networking laboratory and simulation application. It allows the network topology to be built, Cisco devices to be configured and packet movement to be observed without requiring all physical equipment. Cisco describes it as a free simulation tool and virtual lab for networking, IoT and cybersecurity practice.

Wireshark

Wireshark is a packet and protocol analyser. It can capture authorised live traffic, inspect detailed protocol information, apply capture and display filters, save captures and export selected data. These capabilities are suitable for investigating DHCP, DNS, ICMP, TCP retransmissions and other protocol problems.

NetSpot

NetSpot is a wireless discovery, survey and planning application. It can be used for quick Wi-Fi checks, passive and active surveys, channel analysis and wireless coverage visualisation. Its official feature information describes planning, surveys and multiple wireless heatmap visualisations.

TamoGraph

TamoGraph is a professional wireless site survey and RF planning application. It supports passive, active and predictive surveys, spectrum analysis, AP information, heatmaps and detailed reporting. It is suitable for formal verification of wireless performance across building levels.

- FEATURE COMPARISON

| Tool | Primary function | Works with live network data | Main strengths | Main limitations | Selection |
| --- | --- | --- | --- | --- | --- |
| Cisco Packet Tracer | Network design and simulation | No | Builds the complete topology, supports Cisco CLI configuration, Simulation mode, Simple PDU tests and safe failure testing | Simulated results do not prove real throughput, physical signal strength or production performance | Selected |
| Wireshark | Packet capture and protocol analysis | Yes | Detailed packet inspection, powerful filters, protocol statistics and saved capture evidence | Requires authorised access to a suitable capture point; captures may contain sensitive information | Selected |
| NetSpot | Quick wireless analysis and site survey | Yes | Easy wireless discovery, RSSI, SNR, channel and coverage checks; suitable for repeated field measurements | Detailed functions may depend on the edition, device and wireless adapter | Selected |
| TamoGraph | Professional RF survey and planning | Yes, with predictive capability | Passive, active and predictive surveys, formal heatmaps, interference analysis and detailed reports | Commercial licensing may be required and accurate results depend on correct floor plans and calibrated collection | Selected |

- COMPARISON AGAINST PROJECT NEEDS

| Project requirement | Packet Tracer | Wireshark | NetSpot | TamoGraph |
| --- | --- | --- | --- | --- |
| Build and test the full A-Plus topology | Yes | No | No | No |
| Configure Cisco routers and switches | Yes | No | No | No |
| Test VLANs, trunks, SVIs and HSRP | Yes | Observe traffic only | No | No |
| Test Rapid-PVST and redundant uplinks | Yes | Limited observation | No | No |
| Test routing, NAT, ACLs and VPN logic | Yes | Analyse resulting traffic | No | No |
| Capture DHCP, DNS, ICMP and TCP packets | Simulated view | Yes | No | No |
| Diagnose retransmissions or protocol errors | Limited simulation | Yes | No | No |
| Perform quick wireless signal and channel checks | No | Limited | Yes | Yes |
| Produce wireless heatmaps | No | No | Yes | Yes |
| Perform predictive RF planning | No | No | Available depending on platform or edition | Yes |
| Produce formal wireless survey reports | No | Packet capture reports only | Yes | Yes |
| Test without affecting the live network | Yes | Capture only if authorised | Survey activity has low impact | Predictive survey can occur before deployment |

- SELECTED TOOLSET

No single application satisfies every testing requirement. Therefore, the following four-tool combination has been selected.

5.1 Cisco Packet Tracer – primary network simulation tool

Cisco Packet Tracer will be used to:

- Reproduce the Brisbane and Rookwood topology.

- Configure the routers, Layer 3 switches, access switches, servers, WLCs and APs.

- Test Brisbane VLANs 10, 20, 30, 40, 60 and 70.

- Test Rookwood VLANs 110, 120, 130, 140, 160 and 170.

- Verify trunks, SVIs, DHCP relay, HSRP and Rapid-PVST.

- Test NAT, internet access and the site-to-site IPsec VPN.

- Test APlus-Secure and APlus-Guest access rules.

- Simulate a failed uplink and confirm that the redundant path restores connectivity.

- Produce screenshots, CLI output and Simple PDU evidence.

Reason for selection:

Packet Tracer was selected because the project uses Cisco routers and switches and requires the complete proposed network to be modelled safely before physical deployment. It is the main tool for demonstrating the final logical solution.

5.2 Wireshark – primary packet analysis tool

Wireshark will be used to:

- Capture authorised ICMP echo requests and replies.

- Examine DHCP Discover, Offer, Request and Acknowledgement messages.

- Confirm DNS queries and responses.

- Analyse TCP handshakes, resets and retransmissions.

- Identify packet loss, duplicate acknowledgements or delayed responses.

- Confirm whether traffic reaches the expected network interface.

- Observe IPsec-related traffic while recognising that encrypted payload content should not be visible.

- Save filtered packet captures as troubleshooting evidence.

Suggested filters include:

| Filter | Purpose |
| --- | --- |
| icmp | Display ping traffic |
| arp | Display address-resolution traffic |
| dhcp | Display DHCP address-allocation traffic |
| dns | Display DNS queries and responses |
| tcp.analysis.retransmission | Identify TCP retransmissions |
| ip.addr == 10.10.40.10 | Display traffic to or from BRI-SRV-01 |
| esp | Display IPsec Encapsulating Security Payload traffic where visible |

Reason for selection:

Wireshark was selected because Packet Tracer cannot replace detailed analysis of packets from a real network. Wireshark supports investigation of protocol-specific problems and provides saved evidence of actual traffic behaviour.

5.3 NetSpot – quick wireless assessment tool

NetSpot will be used to:

- Detect available SSIDs and APs.

- Record signal strength and signal-to-noise ratio.

- Identify overlapping channels and nearby wireless networks.

- Compare readings near and far from an AP.

- Perform repeatable measurement-point surveys.

- Identify possible weak-coverage areas.

- Produce initial coverage visualisations for the selected school levels.

Reason for selection:

NetSpot was selected because it is suitable for quick, practical wireless checks during the A-Plus site survey and for repeated post-deployment spot checks. It provides simpler field evidence when a complete professional RF model is not required.

5.4 TamoGraph – detailed wireless planning and verification tool

TamoGraph will be used to:

- Import floor plans for the Brisbane levels.

- Model proposed AP placement before deployment.

- Conduct passive and active surveys.

- Produce RSSI, SNR, interference and channel-allocation heatmaps.

- Verify coverage on the two new classroom levels.

- Identify weak areas caused by walls, distance or interference.

- Produce a detailed post-deployment wireless survey report.

Reason for selection:

TamoGraph was selected because the final design requires managed wireless coverage across five Brisbane levels. Its predictive modelling and detailed reporting provide stronger evidence for final AP placement and wireless validation.

- TOOL-TO-TEST ASSIGNMENT

| Test activity | Primary tool | Supporting tool | Evidence |
| --- | --- | --- | --- |
| Build the proposed topology | Packet Tracer | Network diagram | Saved .pkt file and topology screenshot |
| Verify interfaces and VLANs | Packet Tracer | Cisco IOS commands | CLI screenshots |
| Test DHCP and DNS | Packet Tracer | Wireshark | Client configuration and packet capture |
| Test inter-VLAN routing | Packet Tracer | Ping and traceroute | Successful PDU or command output |
| Test guest isolation | Packet Tracer | Wireshark where authorised | Successful internet test and blocked internal test |
| Test the Brisbane–Rookwood VPN | Packet Tracer | Wireshark and router commands | Ping results and IPsec counters |
| Test HSRP and redundant uplinks | Packet Tracer | Continuous ping | Before-and-after failover evidence |
| Conduct a quick Wi-Fi assessment | NetSpot | WLC information | Measurement screenshots and readings table |
| Produce detailed wireless heatmaps | TamoGraph | Approved floor plans | Heatmap and survey report |
| Investigate intermittent packet problems | Wireshark | BRI-NMS-01 historical information | Filtered capture and incident record |

- COMPATIBILITY AND IMPLEMENTATION CONSIDERATIONS

- Packet Tracer is compatible with the Cisco devices used in the simulation, although some advanced IOS commands may be simplified or unavailable.

- Wireshark must be installed on an authorised workstation with access to the relevant network interface or approved capture point.

- NetSpot and TamoGraph require a compatible wireless adapter and accurate measurement locations.

- Floor plans must be correctly scaled before producing predictive or measured heatmaps.

- All tools must be tested on the available school computers before fieldwork.

- Required licences must be approved before using paid functions.

- Testing must follow A-Plus Institute security, privacy and WHS requirements.

- Packet captures must be limited to the required protocols and stored securely.

- Simulation outputs and real measurements must be identified separately in the final evidence.

- FINAL SELECTION DECISION

The selected software combination is:

- Cisco Packet Tracer for topology design, configuration and simulation.

- Wireshark for live packet and protocol analysis.

- NetSpot for quick wireless discovery and site-survey measurements.

- TamoGraph for detailed RF planning, heatmaps and post-deployment verification.

This combination was selected because it covers logical design, configuration testing, packet-level troubleshooting and physical wireless assessment. Together, the tools provide suitable evidence for the A-Plus Institute network while supporting safe testing and structured troubleshooting.