ROUTINE IOS DEVICE MAINTENANCE PLAN AND CHECKLIST

Project: A-Plus Institute Enterprise Network Upgrade
Network Design: Network Option 2 – Final Selected Network
Prepared by: Leonardo Matias Ciarcia
Role: Network Technical Specialist
Date: 6 October 2025
Version: 1.0
Status: Submitted for review

- PURPOSE

This plan defines the routine maintenance activities required for Cisco IOS devices in the A-Plus Institute network. Its purpose is to maintain performance, reliability, security and configuration integrity while reducing the possibility of unplanned outages.

The plan covers the Brisbane five-level campus, the separate Rookwood campus and their connection through the 3TTech ISP and site-to-site IPsec VPN.

- DEVICES COVERED

The plan applies to:

- 3TTECH-ISP where maintenance information is available to the service provider.

- BRI-RTR-FINAL.

- ROO-RTR-FINAL.

- BRI-CORE-L3-A and BRI-CORE-L3-B.

- ROO-CORE-L3.

- BRI-SW-F1 to BRI-SW-F5.

- BRI-SW-SERVER.

- ROO-SW-ADMIN and ROO-SW-CLASS.

- BRI-WLC-01 and ROO-WLC-01 where applicable.

- Associated IOS configurations, routing tables, VLANs, SVIs, trunks, ACLs, NAT, HSRP, Rapid-PVST, SNMP, syslog and VPN settings.

- MAINTENANCE RESPONSIBILITIES

| Role | Responsibility |
| --- | --- |
| Network administrator | Complete daily and weekly checks, review alerts, save backups and maintain the checklist |
| Network technical specialist | Conduct monthly audits, troubleshoot problems and prepare changes |
| Project manager or authorised supervisor | Approve changes that may affect network availability or security |
| 3TTech support | Investigate ISP, WAN and provider-controlled equipment problems |
| Equipment or software supplier | Provide IOS updates, security information and technical support where required |

- MAINTENANCE PRINCIPLES

All maintenance must follow these controls:

- Obtain authorisation before making a configuration or IOS change.

- Review the current topology, IP schema and approved configuration.

- Back up the running and startup configurations.

- Record the device name, current IOS version and change reason.

- Test significant changes in a controlled environment first.

- Complete disruptive maintenance during an approved maintenance window.

- Change only the required settings.

- Maintain console or alternative management access during risky changes.

- Test connectivity, routing, security and redundancy after the change.

- Reverse the change if the acceptance tests fail.

- Save the approved running configuration to startup configuration.

- Update the maintenance register and network documentation.

- ROUTING MAINTENANCE

Network Option 2 currently uses static IPv4 routes, including default and backup paths. Therefore, the primary routing checks are route presence, correct next-hop addressing and destination reachability.

The following commands will be used:

| Command | Purpose |
| --- | --- |
| show ip route | Display the IPv4 routing table |
| show ip route static | Display configured static IPv4 routes |
| show ip protocols | Confirm whether any dynamic routing process is active |
| `show running-config | include ip route` | Review configured IPv4 static routes |
| show ipv6 route | Display IPv6 routes in the controlled dual-stack test environment |
| show ipv6 protocols | Display configured IPv6 routing processes |
| `show running-config | include ipv6 route` | Review configured IPv6 static routes |
| ping | Verify the next hop and destination |
| traceroute | Identify the path taken to the destination |

Because no dynamic routing protocol is part of the approved IPv4 design, an unexpected OSPF, EIGRP or RIP process must be investigated as a possible unauthorised configuration.

If a dynamic routing protocol is approved in the future, neighbour status, route advertisements, authentication, timers and convergence must be added to this maintenance plan.

- ROUTINE MAINTENANCE SCHEDULE

| Frequency | Maintenance task | Devices | Method or command | Evidence |
| --- | --- | --- | --- | --- |
| Daily | Review critical monitoring alerts and syslog events | All critical devices | BRI-NMS-01 and device logs | Daily monitoring log |
| Daily | Confirm Brisbane and Rookwood routers are reachable | Site routers | Ping and NMS status | Availability record |
| Daily | Confirm critical interfaces remain up/up | Routers and core switches | show ip interface brief | CLI output or checklist |
| Daily | Check for repeated interface changes or VPN failure | Site routers and cores | Syslog and VPN alerts | Event record |
| Daily | Review CPU and memory warnings | Routers and core switches | show processes cpu and show processes memory where supported | Health record |
| Weekly | Review IPv4 routes and default paths | Routers and Layer 3 cores | show ip route | Routing-check screenshot |
| Weekly | Review the IPv6 route table if the dual-stack test is active | Test routers and cores | show ipv6 route | IPv6 route output |
| Weekly | Check HSRP active and standby roles | Brisbane cores | show standby brief | HSRP screenshot |
| Weekly | Check Rapid-PVST and blocked redundant links | Brisbane cores and access switches | show spanning-tree | STP screenshot |
| Weekly | Check trunks and allowed VLANs | Core and access switches | show interfaces trunk | Trunk output |
| Weekly | Check interface errors, discards and utilisation | Routers and switches | show interfaces and monitoring records | Interface report |
| Weekly | Check VPN state and traffic counters | Site routers | show crypto isakmp sa and show crypto ipsec sa | VPN output |
| Weekly | Compare running and startup configurations | All managed IOS devices | show running-config and show startup-config | Configuration comparison |
| Weekly | Back up approved configurations | All managed IOS devices | Secure configuration export | Backup register |
| Monthly | Audit VLAN names, IDs and access-port assignments | All switches | show vlan brief | VLAN audit |
| Monthly | Audit SVIs, gateway addresses and helper addresses | Layer 3 cores | Running configuration and interface checks | SVI audit |
| Monthly | Audit ACLs, NAT and guest isolation rules | Site routers and Brisbane cores | show access-lists and NAT checks | Security audit |
| Monthly | Verify SNMP and syslog delivery | Routers and switches | Test monitoring event and inspect BRI-NMS-01 | Monitoring test |
| Monthly | Review administrative access and SSH configuration | Managed network devices | Running-configuration review | Access review |
| Monthly | Review performance and capacity trends | Routers, switches, servers and wireless | Monthly monitoring report | Capacity report |
| Monthly | Verify configuration documentation matches the network | All managed devices | Compare diagram, IP schema and configurations | Documentation review |
| Quarterly | Conduct an approved HSRP and uplink failover test | Brisbane network | Controlled link failure and continuous ping | Redundancy report |
| Quarterly | Test restoration of one backed-up configuration in a controlled environment | Selected device or test device | Approved restore procedure | Restoration report |
| Quarterly | Review Cisco security advisories and IOS update requirements | All Cisco devices | Vendor review | Security review record |
| Quarterly | Review device inventory, serial numbers, warranties and support status | All network devices | Asset register | Updated inventory |
| Six-monthly | Test a proposed IOS update in a supported lab where required | Relevant device model | Lab validation and regression test | IOS test report |
| Six-monthly | Apply an approved IOS update if required | Authorised production device | Change-management procedure | Change and validation record |
| Annually | Complete a full network configuration and security review | Entire network | Technical audit | Annual maintenance report |
| After every change | Save the new approved configuration | Changed device | copy running-config startup-config | Saved configuration evidence |
| After every change | Repeat connectivity, routing, security and redundancy tests | Affected network area | Approved regression tests | Post-change test report |
| After an incident | Review logs, correct the cause and update preventive controls | Affected devices | Incident procedure | Incident closure report |

- CONFIGURATION BACKUP PROCEDURE

Before and after an approved change:

- Record the device hostname and current date.

- Capture show running-config.

- Capture show startup-config.

- Save a secure backup using the following naming convention:

DeviceName_YYYYMMDD_PreChange.cfg
DeviceName_YYYYMMDD_PostChange.cfg

- Store the files in the authorised restricted-access location.

- Confirm that the backup file is readable.

- Record the backup in the maintenance register.

- Do not include passwords, pre-shared keys or SNMP credentials in general screenshots or unsecured reports.

- IOS VERSION AND UPDATE PROCEDURE

IOS updates must not be installed only because a newer version exists. An update must have a documented reason, such as:

- Correction of a known security vulnerability.

- Resolution of a fault affecting the network.

- Requirement for an approved feature.

- Vendor support or compatibility requirement.

Before an update:

- Confirm device model, memory and storage compatibility.

- Read the vendor release notes.

- Back up the configuration and current IOS information.

- Verify the IOS image source and integrity where supported.

- Test the update on equivalent equipment or an approved lab.

- Prepare a rollback plan.

- Schedule the change outside important teaching and administration periods.

- Inform affected stakeholders.

After an update:

- Confirm the device starts correctly.

- Use show version to confirm the installed version.

- Verify interfaces, routes, VLANs, trunks, HSRP, STP, ACLs, NAT and VPN.

- Test staff, student, server, wireless, internet and inter-campus access.

- Monitor the device closely after restoration.

- Record the result and close the change only after successful validation.

Packet Tracer uses simulated IOS features and does not replace compatibility testing on actual production hardware.